## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(4137)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following static characteristics of an instrument: - Accuracy - Resolution - Precision. |  |
| b) | The expected value of the voltage across a resistor is 10 V. However the measurement gives a value of 9.5 V. Calculate - absolute error - % error. |  |
| c) | Explain with an example loading effect of a voltmeter. |  |
| d) | Explain with a diagram the chop method of dual trace generation. |  |
| e) | Define the following characteristics of a pulse: - Rise time - Fall time - Overshoot. |  |
| f) | Describe in brief the construction and working of an LED. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i)Calculate the value of shunt resistance required for using a 50 A meter movement, with an internal resistance of 250, for measuring 0-250 mA. |  |
|  | ii)Calculate the resistance required to use a 0-1 mA meter with an internal resistance of 100 for a 0-50 V meter. |  |
| b) | Draw the circuit of a transistor voltmeter and explain its working. |  |
| c) | Draw a labelled diagram showing the construction of a PMMC mechanism. Explain its working. Write the equation for the developed torque. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of a Wheatstone’s bridge. Derive the equation for balance of the bridge. List one limitation of the bridge. |  |
| b) | Draw the circuit of a Schering bridge. Derive expression for measuring value of unknown capacitor and resistor. |  |
| c) | Draw the circuit of a bridge used for measuring high Q coils. Name the bridge. Derive the expression for unknown inductance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the structure of a cathode ray tube. Write the function of any 6 elements of the CRT. |  |
| b) | Explain with diagrams how the measurement of peak amplitude and frequency is performed using a CRO. |  |
| c) | Draw the block diagram of a Digital Storage Oscilloscope. Explain the digitizing and storing features of the DSO. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw and describe the block diagram of an indirect frequency synthesized signal generator. |  |
| b) | Draw the block diagram of a pulse generator. Explain the working of the basic generating loop. |  |
| c) | Draw the block diagram of a function generator. Explain the generator of triangular and square wave. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a labelled diagram describe the construction and working of an LCD. |  |
| b) | Draw the block diagram of the vertical section of an oscilloscope. Explain each block. |  |

***********