## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-I(4150)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List various frequency bands used for communication systems. |  |
| b) | Define modulation & explain the need for modulation. |  |
| c) | Differentiate between AM and FM. |  |
| d) | Explain how monoaural receiving systems are catered to by stereophonic FM transmitter. |  |
| e) | Explain the cause and effect of Double spotting in an AM receiver. |  |
| f) | Explain the role of Amplitude limiter circuit in FM receiver. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the cause and effects of various types of external noise in communication system. |  |
| b) | With the help of neat waveform, derive the equation of modulation index in terms of maximum and minimum voltages of an AM wave. |  |
| c) | The antenna current of an AM transmitter is 8A, when only carrier is sent, but it increases to 8.93A when carrier is modulated by single sine wave. i)Find the percentage modulation. ii)Determine the antenna current when percentage of modulation changes to 80%. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the percentage power saving when the carrier and one of the sidebands are suppressed in an AM signal modulated to a depth of i) 100% ii) 50%. |  |
| b) | Explain filter method for suppressing a sideband with neat block diagram. State the merits and limitations of the same. |  |
| c) | Explain the frequency spectrum of FM with neat diagram and necessary equations. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Pre-emphasis and De-emphasis in FM system. |  |
| b) | With the help of neat block diagram and vector diagram, explain Armstrong method of generating FM signal. |  |
| c) | Explain the block diagram operation of low level AM transmitter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of basic diode detector circuit. Also state the limitations of the same. |  |
| b) | Explain the operation of phase discriminator circuit. |  |
| c) | Explain the significance of AGC in AM receiver. Also explain how simple AGC is employed in AM receiver. |  |
|  |  | ………2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of Automatic Frequency Control employed in receiver. |  |
| b) | With the help of neat block diagram, explain the operation of Stereophonic FM receiver. |  |

***********