## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC

Subject: Television Engineering(5135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is interlace error? Explain for 16 s delay. |  |
| b) | Define 1) Hue 2) Saturation 3) Luminance. |  |
| c) | List any four advantages of AGC. |  |
| d) | Why Vestigial Sideband Transmission is used in TV transmission? |  |
| e) | Justify the choice of Rectangular frame with aspect ratio 4/3 for TV Transmission and Reception. |  |
| f) | List any three safety precautions in TV servicing. |  |
| g) | List merits of a SECAM-TV system. |  |
| h) | Why picture to sync ratio is kept at 10:4 for TV transmission? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a suitable sketch how video signal is developed in Vidicon camera tube. How does Vidicon differ from Image orthicon? |  |
| b) | Why is scanning necessary in TV system? What are types of scanning carried out? Which method is more efficient and why? |  |
| c) | Explain Horizontal sync in details. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on Yagi-Uda Antenna. What is BALUN? Why is it necessary? |  |
| b) | What is positive modulation and negatives modulation? Why negative modulation is preferred over positive modulation? |  |
| c) | Explain AFC and video detector sections in a TV receiver. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) i) ii) | Explain the following: Automatic Degaussing circuit Grey scale Tracking. |  |
| b) | Explain constructional details of PIL colour picture tube. |  |
| c) | Explain generation of RGB signals. Also explain additive mixing. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List limitations of NTSC system. Explain the sections of a PAL-D colour receiver. |  |
| b) | Explain any five symptoms and respective probable faults in a monochrome receiver. |  |
| c) | Explain the generation of I and Q signals in NTSC. Also, compare NTSC with PAL and SECAM. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Procedure for troubleshooting colour TV receiver |  |
| b) | SECAM Decoder |  |
| c) | Pattern Generator |  |

***********