## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial & Hydro-pneumatic Control(5113)/(511B)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State three points of difference between hydraulic and pneumatic power systems. |  |
| b) | Draw the symbols of the following: |  |
|  | (i) Shuttle valve (ii) quick exhaust valve (iii) check valve. |  |
| c) | Compare meter in and meter out pneumatic circuits. |  |
| d) | In an operator actuated pneumatic circuit, under which circumstances would you use a pilot direction control valve instead of a manually actuated valve. |  |
| e) | List three safety precautions when using pneumatic systems. |  |
| f) | State the functions of FRL unit. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the function of pressure regulating valve, pressure limiting valve and pressure sequence valve. Draw the symbol in each case. |  |
| b) | Explain the construction and working of a 3/2 spool type direction control valve with the help of a neat diagram. |  |
| c) | Draw the block diagram of a hydraulic power supply and explain the function of various blocks. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the symbols of the following types of 5/3 valves: (i) All ports blocked (ii) Outlet to exhaust (iii) Supply to outlet. Explain the function in each case. |  |
| b) | How are components placed in a pneumatic circuit diagram? Illustrate with an example. |  |
| c) | Describe two types of cylinders and state their features. Draw the symbols. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a manually operated pneumatic circuit to control the speed of a double acting cylinder during outstroke operation. |  |
| b) | A double acting cylinder is used in a punching operation. The cylinder is to advance fully when both foot pedal as well as hand operated lever are pressed. The cylinder is to retract automatically. Draw a suitable pneumatic circuit. |  |
| c) | Draw a pneumatic circuit of a fully automatic controlled double acting cylinder. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a time delay valve with the help of an application circuit. |  |
| b) | Explain the application of limit switch in automatic control of cylinder with the help of suitable circuit. |  |
| c) | List the maintenance activities of a pneumatic system. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Discuss the purpose of a hydraulic combination pump. Draw the block diagram of such a pump and explain the functioning. |  |
| b) | Explain the construction of a hydraulic reservoir. |  |
| c) | Compare the hoses and connectors used in pneumatic and hydraulic circuits. |  |

***********