## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Automobile Components(4067)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is factor of safety? List any our factors to be considered for selection of factor of safety. |  |
| b) | A blackboard is fixed to a wall by two screws. If the board weighs 600N, find the size of the screws if the allowable shear stress for screw material is 40 N/mm2. Assume only simple shear load acts on that. |  |
| c) | List any four purposes of using a coupling. |  |
| d) | What is ‘pitch of the spring’ and ‘free length’? |  |
| e) | What is ‘nip’? |  |
| f) | Explain Hooke’s Law. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bracket is subjected to a load of 4.5 KN as shown in fig.1. Determine the required thickness of the section at A-A in order to limit the tensile stress to 70N/mm2. |  |
| b) | A pulley of diameter 300mm is mounted on a shaft at a distance 400mm to the right of left hand bearing as shown in fig.2. The distance between the two bearings is 1000mm. The shaft receives 30 KW power at 600rpm through the pulley and belt from another shaft. Weight of the pulley is 400N. The belt tensions are vertically downward and the ratio of tensions is 3:1. Determine the diameter of the shaft using maximum shear stress theory. Allowable shear stress for the shaft material is 40MPa. |  |
| c) | An overhang crank as shown in fig.3 carries a tangential load of 10 KN at the centre of the crankpin. Find the maximum principal stress and maximum shear stress at the centre of the crankshaft bearing. |  |
|  |  | ….2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cast Iron bracket as shown in fig.4 supports a loaf of 20KN. It is fixed to the horizontal channel by means of 4 identical bolts, 2 at A and 2 at B. The bolts are made of steel 30C8 (Syt=400N/mm2) and factor of safety is 5. Determine the major diameter of the bolts. |  |
| b) | A bracket subjected to an eccentric load of 20 KN is connected to a column by means of 4 identical bolts as shown in fig.5. Determine the size of the bolt if permissible shear stress for the bolt material is 60MPa. |  |
| c) | Design a rigid type flange coupling used to transmit 15 KW power at 300rpm. The design torque is 125% of the rated torque. The shaft keys and bolts are made of plain carbon steel, 40C8 (Syt=300MPa) and factor of safety is 5. Assume Syc=Syt and Ssy=0.5Syt. The flanges are made of grey cast Iron of grade FG200 (Syt-=200N/mm2) and F.O.S=6. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | It is desired to design a square key for fixing a gear on a shaft of 25mm diameter. 15KW Power at 720rpm is transmitted from the shaft to the gear. The key is made of steel having yield point in tension as 460N/mm2 and F.O.S.=3. Assume yield strength in compression equal to yield strength in tension and yield strength in shear is 57.7% of the yield strength in tension. Determine the size of the key. |  |
| b) | Determine the diameter of the hollow push rod made of mild steel of an I.C. engine if maximum force exerted by push rod is 1500N. The length of the push rod is 50 cm. F.O.S. is 2.5 and end coefficient fixity is 2. Take E=210KN/mm2. |  |
| c) | A multidisc clutch has 3 discs on the driving shaft and 2 on the driven. The inside diameter of the contact surface is 120mm. The maximum pressure on the surfaces is limited to 0.1 N/mm2. Design the clutch for transmitting 25KW at 1575 rpm. Assume uniform wear conditions. Coefficient of friction = 0.3. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a close coiled helical spring for service load ranging from 2300N to 2700N. The axial compression of the spring for that range is 7mm, spring index is 5, allowable shear stress= 400 MPa, G=83KN/mm2. |  |
| b) | A locomotive semi-elliptical laminated spring has an overall length of 1m and sustains a load of 70KN at its centre. The spring has 2 extra full length leaves and 15 graduated leaves with a central band 100mm wide. All the leaves are to be stressed to 400MPa when fully loaded. The ratio of total spring depth to that of width is 2. Determine the thickness and width of leaves. Take E=200KN/mm2. | ….3/- |
|  | -3- |  |
| c) | The connecting rod of a four stroke diesel engine is made of M.S. It is of circular section and of length 550mm. The diameter and stroke of cylinder are 150mm and 240mm respectively. The maximum combustion pressure is 4.7N/mm2. Determine the diameter of the rod to be used for a factor of safety of 3 with a material having yield point of 330N/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A single disc dry friction clutch plate effective on both sides is to be designed for a car, powered by an engine, which develops a torque of 117 N-m at 2500 rpm. Calculate the outside and the inside diameters of the friction facing, given that the coefficient of friction=0.35 and permissible unit pressure on friction facing is 0.25 MPa. Assume inside diameter of facing as 0.55 times the outside diameter. |  |
| b) | A trunk type of C.I piston for I.C engine has a diameter of 100mm and length 160mm. Maximum explosion pressure = 4MPa, Maximum permissible tensile stress for Piston head Cast Iron= 30MPa and that for piston pin material is 42MPa. Bearing pressure for piston pin should not exceed 7MPa. Design - The thickness of the head on the basis of strength - Dimensions of the hollow piston pin. |  |
| c) (i) | Explain the general consideration for selection of material and manufacturing process. | (3) |
| (ii) | List the general design considerations. | (3) |