## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Computer Engg.

Subject: Computer Organisation-I(4220)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the difference between CPU bus ands System bus. |  |
| b) | What is hardwiring? |  |
| c) | What techniques are used to increase the speed of operation in a CPU? |  |
| d) | List and illustrate with suitable examples the basic symbols for Register transfer. |  |
| e) | Subtract in two’s complement representation (i)(111000)-(110011) (ii) (11000011)-(11101000) |  |
| f) | What are the characteristics of Multiprocessors? |  |
| g) | Represent the following two’s complement value in decimal: i) 1101011 ii) 0101101 |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of Stored Program Control. |  |
| b) | Explain the difference between single and multiple bus architecture. |  |
| c) | Explain the role of the following Registers in the CPU: - Accumulator - Program Counter - Instruction Register - Stack Pointer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the algorithm and draw Block diagram of the hardware to be used for Two’s complement addition and subtraction. |  |
| b) | Using an appropriate division algorithm divide-145 by 13 in binary two’s complement rotation using 12 bit words. |  |
| c) | Express the following in IEEE 32 bit floating point format: i) -1.5 ii) 354 iii)-65. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following addressing modes: i) Register ii) Immediate iii) Indirect iv) Direct. |  |
| b) | Explain the events that take place during the first 3 states of a Microprocessor Machine Cycle. |  |
| c) | Explain Microprogramming. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are shift Micro operations? Explain with an example. |  |
| b) | What are logic Micro operations? Explain the implementation with a logic diagram. |  |
| c) | Explain one byte, two byte and 3 byte instructions. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Hardwiring |  |
| b) | Booths Algorithm |  |
| b) | Types of Buses |  |
| c) | Arithmetic Micro operation |  |
| d) | Instruction cycle |  |
| e) | Stored Program Computer |  |