## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Electric Drives(4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is multi motor drive? Give one example of such drive. |  |
| b) | Define steady state stability of a drive. Also state other types of stabilities in drive. |  |
| c) | State the three advantages of electrical braking over mechanical braking. |  |
| d) | Define (1) Heating time constant (2) Cooking time constant of electric drive. |  |
| e) | Give two points of distinction between Limit switch and float switch. |  |
| f) | State the types of motors used in lathes and justify your choice. |  |
| g) | State the types of drives used in sugar industries and textile mills. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the significance of joint speed torque characteristic of electric motor & driven unit? Also, draw such characteristic for any drive of your choice and explain it in brief. |  |
| b) | State the different types of loads based on variation of load torque with time to be driven by electric motor. Also state at least two examples of each load. |  |
| c) | Define (1)Referred load torque (2)Referred moment of inertia. |  |
|  | Also state the expression of each and indicate each term used in such expressions. |  |
| d) | A horizontal conveyor belt moving at a uniform velocity of 1m/sec, transports load at a rate of 50,000 Kg/hour. The belt is 180m long and is drive by 960 R.P.M. Determine the equivalent rotational inertia at the motor shaft. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify electrical braking used for drive. Which of the braking is most efficient, semi-efficient and inefficient giving reasons for your choice. |  |
| b) | With the help of neat circuit diagram and speed v/s torque characteristic explain in brief dynamic braking in d.c. shunt motor. |  |
| c) | What is slip power? Explain in brief with the help of neat diagram slip power recovery control. What purpose does it serve? |  |
| d) | How can constant torque operation of constant horse power operation be achieved during speed control of 3 phase squirrel cage Induction motor by using pole changing method .Explain in brief with help of neat connection diagram of stator as well as speed v/s torque characteristic. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out all the classes of duty we come across in electric motor. Also state one application of any 4 classes of duty of your choice. |  |
| b) | Explain in brief the effect of environmental factors on selection of rating of electric motor. |  |
| c) | What do you mean by load equalisation? What is its necessity? How can it be achieved in practice? Hence explain in brief. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of control circuit indicating the reversal of direction of rotation of motor by using direct method and explain it in brief. |  |
| b) | State the types of sequential control operations of motor and explain sequential control operation of motor with time delay, with the help of control circuit diagram. |  |
| c) | Explain in brief remote control operation of three phase Induction motor with help of neat labelled power circuit diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the type/s of motor/s used for following applications and justify your choice: (1)Pumps (2)Machine Tools. |  |
| b) | What is the function of selector switch? State their types & draw their diagrams. Is it different from proximity switch, giving reasons for your choice. |  |
| c) | Give three points of distinction between: |  |
|  | (1)Electromagnetic contactor & overload relay |  |
|  | (2)Types of Push buttons w.r.t. their symbols, types of contacts used & one application. |  |

***********