## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC/EI/IS

Subject: Electronic Measurements(4137) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw and describe the structure of an LCD. |  |
| b) | Draw an LED seven-segment display and describe it. |  |
| c) | Which are the three types of errors in measurement? |  |
| d) | Describe the time standards used. |  |
| e) | What is a direct-synthesis type of signal generator? |  |
| f) | Define sensitivity of a voltmeter. What is the sensitivity of a voltmeter, which uses a PMMC of internal resistance of 50 and full-scale deflection current of 1mA. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram and design an Ayrton shunt-type ammeter to provide current ranges of 1A and 10A. A d’Arsonval movement with an internal resistance of 50 and full-scale deflection current of 1mA is to be used. |  |
| b) | Draw the diagram of an FET voltmeter and describe its operation. |  |
| c) | Draw a series-type ohmmeter and derive the expression for the series resistor R1 in terms of the half-scale deflection resistor Rh. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the Wheatstone’s bridge shown in fig.1, calculate the current through the galvanometer G, whose internal resistance is 100. |  |
| b) | Draw the circuit diagram of the Schering’s bridge and describe mathematically, how the unknown parameter is calculated. |  |
| c) | A balanced AC bridge has the following constants: Arm AB: R = 1000 in parallel with C = 0.5F Arm BC: R = 1000 in series with C = 0.5F Arm CD: L = 30mH in series with R = 200 Arm DA: unknown. The frequency of oscillation is 1000Hz. Find the constants of the arm DA. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe a CRO with the help of its complete block diagram. |  |
| b) | Draw the circuit diagram of a triggered-sweep generator in a CRO and explain how it produces a sweep, with the help of waveforms. |  |
| c) | Distinguish between a dual-beam and dual trace CRO with the help of a block diagram of each. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the of a neat block diagram describe a wide band sweep generator. |  |
| b) | Draw the block diagram of an indirect-synthesis type of function generator and describe its operation. |  |
| c) | Draw and explain the linearising circuit used in a sweep generator. | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Describe in brief, the electrostatic focusing used in a CRO. |  |
| b) | Describe the use of Lissajous patterns. Draw any 3 Lissajous patterns. |  |
| c) | For a measuring instrument, describe: i) Accuracy ii) Resolution. |  |
| d) | Describe in brief, a bar graph display, with the help of a diagram. |  |

***********