## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | What do you understand by equivalent section? Explain how do you obtain the equivalent section. |  |
| b) | Explain the following: i) Middle third rule ii) Kernel for different sections. |  |
| c) | What is shear connector? Explain how will you find the spacing of the same. |  |
| d) | Sketch the Mohr’s circle corresponding to the case of pure shear stress at a point in a stressed material. |  |
| e) | A fixed beam of span ‘L’ carries an Udl over left half of the span. Sketch SFD and BMD for the beam. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A wooden beam 150 mm wide and 300 mm deep attached with steel plat of 12 mm thick, 150 mm wide @ top and bottom. If stress produced in steel is 60 N/mm2, find the safe udl the beam can support over an effective span of 4 m. Take ES = 16 EW. |  |
| b) | A column section 500 mm x 300 mm is subjected to an eccentric load of 250 KN. Eccentricity of load is 30 mm below XX–axis and 50 mm towards right of YY-axis. Calculate stress intensities at four corners of column. |  |
| c) | Locate ‘Kernel’ of following sections: i) Circular column of 600 mm diameter ii) Hollow rectangular section with internal dimension 400mm x 200 mm, having 50 mm thickness alround. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a certain point in a strained material, normal stresses of 125 N/mm2 compression and 50 N/mm2 tensile acts. Find the resultant stress across a plane making an ample of 40 to the major stress. Use Mohr’s circle method. |  |
| b) | A tension member is formed by connecting two wooden members 250 mm wide x 400 mm deep. Angle of inclination made by the surface of content is 45. If the maximum normal and shear stresses @ the joint are 7.5 N/mm2 and 2.5 N/mm2 respectively, determine safe value of the tensile load. |  |
| c) | At a point in a strained material, the normal stresses on two planes at right angles to each other are 20 N/mm2 and 10 N/mm2 both compressive. There is also shear stress of 5 N/mm2 on these planes. Calculate the maximum shear stress at this point. Also locate the plane for which the resultant stress has maximum obliquity. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A fixed beam of 7 m span carries an udl of 45 KN/m for a length of 3.5 m placed symmetrically in the middle of beam. Find the fixed end moments and sketch SFD and BMD. | …….2/- |
|  | -2- |  |
| b) | A cantilever beam 4.5 m span carries an udl of 25 KN/m throughout the span along with a point load of 30 KN at 2 m from free end. Find the maximum slope and defection for the beam. Take E = 2 x 105 N/mm2, Ixx = 3500 x 104 mm4. |  |
| c) | A simply supported beam 300 mm wide and 500 mm deep carries point load of 25 KN, 40 KN and 30 KN at distances of 1.5m, 3.0m and 4.5m from left end support, the span of beam is 6m. Determine slope at both supports and deflection under each point load. Take E = 0.2 x 105 N/mm2. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | A beam ABC 6m long is fixed at A, simply supported at B and free at C. Span AB=4.5 m and overhang BC = 1.5 m. The beam AB carries an udl of 20 KN/m and 35 KN load at free end ‘C’. Draw SFD and BMD. Use moment distribution method. | (9) |
|  | ### OR |  |
| b) | A portal frame ABCD with supports A and D fixed, carries an udl of 30 KN/m over the entire horizontal span BC = 4.5 m. Height of columns AB=CD=4.0 m. If moment of inertia of beam is ‘2I’ and columns 1.5I each, draw BMD and SFD. | (9) |
| c) | Explain the terms: (i) Unbalanced moment (ii) Distribution factor. | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cast iron column of hollow circular section has to transmit a load of 500 KN. Calculate the external diameter, if the length of column is 5m and both ends are fixed. The thickness of metal is 20 mm. Use Rankine’s formula. Take c=550N/mm2 and a factor of safety as ‘5’. |  |
| b) | In an experimental determination of buckling load for 12 mm diameter mild steel, pin-ended struts, following observations were made: i) Length 300 mm, load 10.5 KN ii) Length 500 mm load 7.6 KN. Verify which of above two values conforms with the Euler’s formula for the critical load. |  |
| c) | An I section is used as steel stanchion. The height of column is 5m with one end fixed and other end hinged. Find the safe load using Rankine’s formula. Take c = 300 N/mm2. Rankine’s constant  =1/7500 and factor of safety ‘3’. Properties of I section flanges 250 mm wide and 10 mm thick each, web 230 mm deep and 8 mm thick. |  |

***********