## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Engineering Metrology & Quality Control(ME406) [Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | If the combination of slip gauges used with a sine bar of length 100mm between centres of rollers is 42.54mm, calculate the angle value. |  |
| b) | Draw a sketch of optical profile projector and describe how it is used. |  |
| c) | Sketch a bell shaped curve and explain its relevance to six sigma concept. |  |
| d) | Explain the term ‘total quality control’. |  |
| e) | Illustrate and explain the difference between a thread micrometer and a normal micrometer. How can it be used for thread measurement? |  |
| f) | For designation of a hole tolerance, H7 is used with i = 1.3066 m, IT7=16i and basic size as 30mm. Compute the hole size (maximum and minimum). |  |
| g) | What are the advantages and disadvantages of Mechanical comparators? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With regard to any measuring instrument, explain these terms involved: i) Least count ii) Unit of Measurement iii) Zero error iv) Sensitivity v) Magnification vi) Precision. |  |
| b) | What is the necessity of allowances and tolerances in guage design? What are the generally used values for them? |  |
| c) | Compare Hole & haft basis system of tolerance designation. (Give minimum 3 points) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the principle of Vernier Caliper. Explain briefly the construction of Vernier Caliper. |  |
| b) | Using standard set of 13 pieces angle gauges, give the combination set required to get: i) 103-8’-42” ii) 35-12’-12”. |  |
| c) | Draw and explain the working of a ‘Sigma’ comparator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) In screw thread terminology, what is meant by M14 x 1.5? |  |
|  | ii) Explain any one method of measurement of major diameter of screw thread. |  |
| b) | Write a note on errors in gear measurement. |  |
| c) | Explain with a neat sketch the working of Tomlinson surface meter. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Discuss the economic aspects of ‘quality of design’. |  |
| b) | Sketch a p-chart after computing UCL & LCL for the fraction defectives for an Industry producing bearings as given below: |  |
| Day; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. Fraction Defective; 0.112; 0.058; 0.062; 0.091; 0.004; 0.035; 0.087; 0.099; 0.075; 0.042. |
| c) | Explain: i) single sampling plan ii) Double sampling plan. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | ISO system implementation |  |
| b) | Quality audit |  |
| c) | Squareness testing of spindle axis with table |  |
| d) | Use of Pneumatic Comparator to check bore of cylinders |  |
| e) | Gear tooth Vernier |  |
| f) | Wavelength standard |  |

***********