## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Manufacturing Process-I(4034) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a note on permanent mould casting. |  |
| b) | Give an example of any six products which are manufactured by hot rolling process. |  |
| c) | Write short note on self lubricating bearing. |  |
| d) | List different core boxes. |  |
| e) | What do you understand by the term ‘core’ with respect to moulding? How is it manufactured? |  |
| f) | What are the safety precautions to be observed in welding? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a simple labelled sketch of equipment setup used for manual metal Arc welding. List the various power sources which can be used. |  |
| b) | List and sketch various positions of welding for Plate and Pipes for fillet weld and groove weld. |  |
| c) | With the help of a sketch explain the three types of flames which can be obtained in gas welding. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With the help of a sketch explain any two moulding methods based on the process. |  |
| b) | List various properties of moulding sand and explain any two. |  |
| c) | List the various molten metal handling equipments and sketch any two. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | List any six types of defects in casting and state the causes and remedies. |  |
| b) | What do you mean by pattern making allowance? List various allowances and explain any two in brief. |  |
| c) | Draw a neat sketch of: i) Loose Piece Pattern ii) Sweep Pattern. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the process of hot extrusion with the help of sketches. |  |
| b) | List any six differences between ‘Hot working’ and ‘Cold working’ of metals. |  |
| c) | What are the various steps involved in manufacturing of metal powder? |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Sand preparation |  |
| b) | Melting furnaces used for non–ferrous metals |  |
| c) | Welding electrodes |  |
| d) | Cold extrusion |  |
| e) | Products of Powder metallurgy |  |

***********