## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | COP of the heat pump is greater than the COP of the refrigerator operating between identical temperatures. Why? |  |
| b) | How are the refrigerants designated? |  |
| c) | Explain the process of subcooling using T-S plot. |  |
| d) | State the functions of following components in vapour absorption system: i) Generator ii) Analyser. |  |
| e) | Explain in brief the working of a centrifugal compressor. |  |
| f) | Draw a neat labelled sketch of psychrometric chart. |  |
| g) | Represent the mixing of two air streams on the psychrometric chart. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving neat sketch explain the vapour compression cycle. |  |
| b) | Explain, giving neat sketch, the construction and working of Electrolux refrigerator. |  |
| c) | With the help of neat sketch explain the cascade system of refrigeration. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & working of the following giving neat sketches: i) Capillary tube ii) Finned tube evaportor. |  |
| b) | With the help of a neat sketch explain the construction and working of thermostatic expansion valve. |  |
| c) | Define the following: i) Wet bulb temperature ii) Specific humidity iii) Relative humidity iv) Sensible heat. |  |
| d) | Explain the construction & working of following components giving neat sketches: i) Air washer ii) Filter. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | One Kg of air at 40C dry bulb temperature and 50% relative humidity is mixed with 2 Kg of air at 20C dry bulb temperature and 20C dew point temperature. Calculate the temperature and specific humidity of the mixture. |  |
| b) | Represent the following processes on the psychrometric charts: i) Sensible heating ii) Sensible cooling iii) Cooling & humidification iv) Heating and dehumidification. |  |
| c) | Write a note on Limit switches. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write down the salient features of the duct system. |  |
| b) | List down the requirements of a good room air distribution system. |  |
| c) | Giving neat sketch explain the construction & working of an ice plant. |  |
| d) | Write a note on air distribution systems. | ……..2/- |
|  | -2- |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Window air conditioner |  |
| b) | Domestic refrigerator |  |
| c) | Automatic humidity control |  |
| d) | Room Sensible Heat Factor (RSHF) |  |
| e) | Solenoid valve |  |
| f) | Desirable properties of an ideal refrigerant |  |

***********