## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engineering(4017)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief: flow and lift irrigation, direct and storage irrigation, perennial and inudation irrigation. |  |
| b) | What is meant by runoff? What are the factors affecting runoff? Give any one formula to compute runoff. |  |
| c) | What are the factors affecting duty of water? List out any 5 points. |  |
| d) | What is bandhara irrigation system? Draw a neat section and plan of a bandhara. |  |
| e) | Draw a neat c/s of a typical irrigation canal partly in cutting and partly in embankment. |  |
| f) | List out any 5 preliminary investigations to be conducted before taking up an irrigation project. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw and explain in brief the working of an automatic type of any raingauge. |  |
| b) | Explain with a sketch the method of computation of average rainfall by Theisson’s polygon method. |  |
| c) | What is hydrologic cycle? Explain with a neat sketch. |  |
| d) | Define: Duty, Delta and Base period. Also state their units of measurement. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A watercourse commands an irrigated area 1500ha. The intensity of irrigation of rice in this area is 70%. The transplantation of rice crop takes 15 days and the total depth of water required by the crop on the field is 500mm. During this period, the useful rainfall on the field is 120mm. Find the duty of irrigation water for the crop on the field during transplantation at the head of the field and at the head of the watercourse. Assume losses of 20% in the watercourse. Also calculate discharge required in the watercourse. |  |
| b) | - Define : Intensity of irrigation, Gross command area, Capacity factor, Outlet factor, Time factor, cumecday. - Differentiate and explain clearly: L.S.L., M.W.L., H.F.L. in respect of a storage reservoir. |  |
| c) | List out any 6 points to control sedimentation in reservoirs. | ………2/- |

|  | -2- |  |  |
| --- | --- | --- | --- |
| Q.No.4. | Answer any two of the following Questions: |  | 2 x 6 = 12 |
| a) | Fix the control levels of a reservoir using the following data: C.C.A. = 150ha.m., Gross duty = 250 ha/Mm3, Dead storage at LSL = 18 ha.m., Tank losses = 15 % of crop water requirement, Free board = 2.0m. |  |  |
| Contour (R.L. in m); 15; 25; 50; 75; 100; 150; 200; 250; 300; 350; 375. Capacity (in Ha.m); 10; 22; 36; 52; 65; 73; 98; 125; 150; 160; 180. |
| b) | Draw a neat section of a gravity dam. Show all the forces acting on it with their expressions. Establish the conditions for ‘No sliding’ and ‘No tension at base’ – in terms of base width, specific gravity of water, friction factor, height of dam, etc. |  |  |
| c) | - Enlist any 6 advantages of a gravity dam over earth dams. - Write short notes on galleries in gravity dam. |  |  |
| Q.No.5. | Answer any four of the following Questions: |  | 4 x 3 = 12 |
| a) | List out any 3 types of spill ways and draw neatly each. |  |  |
| b) | What are the energy dissipators? Show the types with sketches. |  |  |
| c) | Draw simple sketch of the three types of earth dams. |  |  |
| d) | Draw and explain rock toe and horizontal filter type of drainage in earth dams. |  |  |
| e) | What are the advantages and disadvantages of Bandhara system of irrigation? |  |  |
| f) | Write short notes on any 3 types of canal lining. |  |  |
| Q.No.6. |  | Write short notes on any four: | 4 x 3 = 12 |
| a) |  | List out any 6 important advantages of irrigation system. |  |
| b) |  | Differentiate weirs and barrages. Draw sketches of a typical weir and a barrage of any one type. |  |
| c) |  | Explain in brief an aqueduct, a super passage and level crossing. |  |
| d) |  | Make list of any 6 points on final investigation and preliminary preparatory works/drawings before starting an irrigation project. |  |
| e) |  | Write short notes on maintenance and repairs of a gravity dam. |  |

***********