## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define and explain transients in electrical circuits. |  |
| b) | In a simple saw-tooth generator circuit, the thyratron switches on at 150V and switches off at 10V. If this circuit is supplied with 250V dc source, find the time period of the sawtooth wave. |  |
| c) | State and explain Kirchoff’s current law. |  |
| d) | Refer to fig.1 and answer the following: i) What is the phase angle value? ii) What is the value of power factor? iii) Is current leading or lagging? |  |
| e) | State and explain the conditions for balance of ac bridge circuit. |  |
| f) | The arms of a four arm bridge ABCD, supplied with sinusoidal voltage have the following values. AB: 200 resistance in parallel with 1F capacitor; BC: 400 resistor; CD: 1000 resistor and DA: Resistance R in series with a 2F capacitor. Determine i) the value of R and ii) the supply frequency at which the bridge will be balanced. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Using nodal voltage method, find the magnitude and direction of current I of fig.2. |  |
| b) | State Thevenin’s theorem. For the circuit shown in fig.3, determine the current through RL when its value is 50. |  |
| c) | Find the resistance at the A-B terminals in the electric circuit shown in fig. 4 using /Y transformation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pure resistance of 50 is in series with a pure capacitance of 100F. The series combination is connected across 100V, 50Hz supply. Find i) Impedance ii) Current iii) Power factor iv) Phase angle v) Voltage across resistor vi) Voltage across capacitor. |  |
| b) | Two impedances Z1= (8+j6) and Z2 =n (3-j4) ohms are in parallel. If total current of the combination is 25A, find the current taken and power consumed by each branch. |  |
| c) | In a circuit the equations for instantaneous voltage and current are given by v = 141.4 sin (wt - 2/3) volts and i = 7.07 sin (wt - /2) amps, where w = 314 rad/sec. Find i) Phase angle ii) rms value of voltage and current iii) Impedance iv) average power v) instantaneous power at the instant t=0 and vi) power factor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | An RLC series circuit is connected to a 20V variable frequency supply. If R = 20, L = 20mH and C = 0.5F, calculate the following: i) Resonant frequency fo ii) Resonant circuit Qo iii) Half power bandwidth and iv) Maximum power dissipated at fo. | ………2/- |

-2-

| b) | An inductive circuit of resistance 2 and inductance 0.01H is connected to a 250V, 50Hz supply. What capacitance placed in parallel will produce resonance? Find the total current taken from the supply and the current in branch circuits. |  |
| --- | --- | --- |
| c) | Briefly explain the graphical representation of RLC series circuit. Draw the necessary diagrams. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Similar coils A, B and C are available. Each coil has 9 resistance and 12 reactance. They are connected in delta to a 3-phase 440V, 50Hz supply. Calculate for this load: i) the line current ii) the power factor iii) the total Kilovolt amperes and iv) the total Kilowatts. |  |
| b) | A three phase load consists of three inductive coils, each of resistance 50 and inductance 0.3H. The supply is 415V, 50Hz. The load is connected in star. Calculate i) the line current ii) the power factor iii) the total active power and iv) the total apparent power. |  |
| c) | A delta connected balanced three phase load is supplied from a 3-phase, 400V supply. The line current is 20A and the power taken by the load is 10,000 watts. Find i) impedance of each branch ii) the line current, power factor and power consumed if the same load is connected in star. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Determine the values of the symmetrical components of a system of currents. IR = (0+j120)A, IY = (50-j100)A and IB = (-100-J50)A. Phase sequence is RYB. |  |
| b) | In a 3 phase, 4 wire system, the currents in the RY and B lines are as follows: IR =10030º; Iy = 50300º and IB = 30180º. Calculate the positive, negative and zero-phase sequence currents. |  |
| c) | Draw the circuit diagram of Hay’s bridge and give the expressions for unknown quantities at balance. What are its advantages and disadvantages? |  |