## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Pole pitch ii) Front pitch iii) Back pitch. |  |
| b) | State the causes of failure of a d.c. generator to build up the voltage. |  |
| c) | What is the necessity of parallel operation of d.c. generators? What is a busbar? |  |
| d) | Define: i) Mechanical Efficiency ii) Electrical Efficiency iii) Overall Efficiency of d.c. motor. |  |
| e) | Give three points of distinction between d.c. shunt motor and d.c. series motor w.r.t. its starting torque, variation in speed & two applications. |  |
| f) | Define armature reaction in d.c. generator. Also state its effects. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the diagram of simple Loop generator and explain its working principle in brief. |  |
| b) | Give six points of distinction between Lap winding & Wave winding. |  |
| c) | Classify the d.c. generators. Also state the e.m.f. equation for any one of such generator of your choice. Also state current equation for such generator and indicate each term used in expression. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a neat diagram of a third brush d.c. welding generator and explain its construction. |  |
| b) | Explain in brief the working of a bipolar welding generator and draw its voltage current characteristic. |  |
| c) | A 4 pole dc. shunt generator with a Lap wound armature has an armature resistance of 0.1 ohm and field resistance of 50 ohms. The generator is supplying a total load of 2.4 KW. Determine i) armature current ii) Generated e.m.f. given that brush contact drop is 1V per brush. |  |
| d) | A d.c. shunt generator gives full load output of 30 KW at a terminal voltage of 200V. The armature and shunt field resistances are 0.05 and 50 ohms. The Iron and friction losses are 1000 watts. Calculate: i) Generated e.m.f. ii) Copper losses iii) Efficiency of generator. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the connection diagram of d.c. shunt generator to be connected in parallel to the busbar to which another generator is already connected. Also, state the steps involved in the procedure of such parallel operation. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | Two 220V d.c. generators, each having linear external characteristics operate in parallel. One machine has a terminal voltage of 270V at no load and 220V at a load current of 35A. The second machine has a voltage of 280V at no load and 220V at 50A. Determine the output current of each machine as well as bus bar voltage if total load is 60A. Also, determine KW output of each machine. |  |
| c) | Draw the O.C.C. curve of a d.c. series generator and explain it in brief. Also draw its external characteristics. |  |
| d) | Which of the d.c. generators are most suitable for parallel operation? Give reason for your choice. What is the function of equalizer bar? |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain in brief the principle of operation of d.c. motor with the help of neat diagram. By which rule can you obtain direction of rotation of such motor? Hence state such rule in brief. |  |
| b) | State the methods used for speed control of d.c. series motor and explain any one of such method of your choice with the help of neat diagram. |  |
| c) | “A d.c. motor of 5 H.P. capacity should not be started directly on line”. Say true or false and justify your choice. |  |
| d) | Draw the mechanical characteristics of following motors: i) D.C. series motor ii) D.C. shunt motor iii) D.C. cumulative compound motor. Also, state the factors on which speed of motor depends. Hence, state the expression for speed of motor. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the methods used for improving commutation in d.c. machine. Explain any one of such method of your choice with the help of neat diagram. |  |
| b) | State the different types of losses we come across in d.c. motor. State the device used for starting of a d.c. series motor. Also draw its neat diagram. |  |
| c) | A 120V d.c. shunt motor has an armature resistance of 0.2. And field resistance of 60. It runs at 1800 r.p.m. when taking full load current of 40A. Find the speed of motor, when operating with half load. |  |

***********