## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg.

Subject: Construction Advanced-III(5007)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain use of geotextile in reinforced earth constructions. |  |
| b) | Explain various methods of detecting ground water table. |  |
| c) | Draw a neat sketch of North light roof truss. |  |
| d) | What do you mean by short-crete? Explain. |  |
| e) | What are the objects and requirements of demolition? |  |
| f) | Bring out the difference between formwork or false work. |  |
| g) | What are portal frames? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write down the various methods used for lining of chimneys and explain field procedures. |  |
| b) | Explain with a neat sketch the push launching method of construction of a bridge. |  |
| c) | Explain grouting & guniting methods. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain electrical resistivity method with a neat sketch. |  |
| b) | Draw a neat figure of shell roofs and write down advantages & disadvantages of them. |  |
| c) | Define claddings. Explain function & requirements for cladding works. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention the various pre-fabricated units used for construction. Explain the construction details of any one. |  |
| b) | What are mastics & sealants? Explain the application procedure of them. |  |
| c) | Write down various safety procedures to be taken during demolition of structures. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain the areas of application of reinforced earth construction. |  |
| b) | With a neat sketch explain field procedure of constructing reinforced earth wall. |  |
| c) | Explain with a neat sketch perimeter trench method of excavation of deep trenches. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Yield of wells |  |
| b) | Continuous deck construction of bridges with movable forms |  |
| c) | Slip forms |  |
| d) | Tremie concreting |  |
| e) | Hand demolitions |  |

***********