## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the principle of release of energy by nuclear reaction. |  |
| b) | Why are cooling towers used in Power Plants? |  |
| c) | Give the advantages & disadvantages of hydro-electric Power Plant. |  |
| d) | What are the functions of feed water loop? |  |
| e) | What factors are taken into consideration while selecting a site for thermal Power Station? |  |
| f) | What are the advantages of Gas Turbine Power Plant over Diesel Engine Power Plant? |  |
| g) | What is meant by load curve? What is its significance in power generation? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Power Plant supplies the following loads to the consumers: Time in Hrs; 0-6; 6-8; 8-12; 12-14; 14-18; 18-22; 22-24. Load (MW); 40; 50; 60; 50; 80; 100; 30. Draw the load curve and find - Energy generated by the Plant in a day - Average load on the Plant - Load factor. |  |
| b) | Draw a neat diagram of storage type hydro-electric Power Plant and describe the functions of each component used in the Plant. |  |
| c) | Draw the layout of a diesel engine Power Plant and explain it briefly. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the working of open cycle gas turbine Power plant. |  |
| b) | With a neat sketch explain pulverised coal handling system. |  |
| c) | Explain the construction and working of Electrostatic Precipitator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the general layout of a coal based Steam Power Plant. |  |
| b) | Explain why coal needs to be prepared. What are the various stages in the preparation of coal? |  |
| c) | Explain briefly the zeolite process with a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the construction and working of induced draft cross flow type cooling tower. |  |
| b) | With a neat sketch explain the construction and working of Boiling Water Reactor. | …….2/- |
|  | -2- |  |
| c) | Discuss the radiation hazards and the methods used for radioactive disposal. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Effect of variable load on Power Plant |  |
| b) | Engine cooling system in I.C Engine Power Plant |  |
| c) | Mechanical dust collectors |  |
| d) | Demineralisation process |  |
| e) | Classification of hydro-electric Power Plants |  |
| f) | Prevention of carry over losses from cooling tower |  |

***********