## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(ME401) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw stress v/s strain curve for ductile material and show proportionality limit, elastic limit, yield point, ultimate stress and breaking stress on it. |  |
| b) | What are Principal planes? How do the stresses occur on the principal planes? |  |
| c) | What conclusion do we make by looking at shear force diagram if: i) Zero or no change in sign ii) A straight line parallel to reference line iii) Inclined straight line. |  |
| d) | Define “Moment of Inertia”. |  |
| e) | Write the bending equation on the theory of simple bending & explain the various parameters involved. |  |
| f) | What are the assumptions made in Theory of pure Torsion? |  |
| g) | What is ‘point of contraflexure’? What is its importance to the engineer? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel rail is 6m long and is laid at a temperature of 27C. The maximum temperature expected is 41C. Take  = 12 x 10-6/C, E = 2 x 105 MPa. - Determine the minimum gap between two rails to be left so that temperature stresses do not develop. - Determine the thermal stresses developed if no gap is provided between the rails. - If a gap of 1.5mm is provided for expansion. |  |
| b) | A steel bar ABCD has length AB 250mm in length and 28mm in diameter. Length BC is 450mm in length and is square cross-section of 30mm side and length CD is 200mm long having 15mm diameter. The bar is subjected to an axial compressive load, which produces a stress of 40 MPa on the largest cross section. Determine the total decrease in length when the load is applied. Take E = 2 x 105 N/mm2. | ……2/- |
|  | -2- |  |
| c) | A steel bar 1m long, 50mm wide and 20mm deep is subjected to a pull of 170KN in the direction of its length. Find the change in length, width and thickness of bar. Also find volumetric strain. Take E = 200 GPa and  = 0.3 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The state of stress at a point in a strained material is shown in fig.1. Determine i) The magnitude of principal stress ii) The direction of principal planes iii) The magnitude of maximum shear stress. |  |
| b) | For an element under stresses x=100Mpa (tensile), y=0, shear stress xy = 20Mpa. Find analytically principal stresses & maximum shear stress. Also find obliquity of major principal plane to the horizontal axis. |  |
| c) | Determine the moment of Inertia of an I section about its horizontal centroidal axis. The dimensions of I-section are Top flange : 300 x 25 mm, Bottom flange : 180 x 20 mm, Web: 300 x 30 mm. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw shear force and bending moment diagrams for the cantilever shown in fig.2. |  |
| b) | Draw shear force and bending moment diagram for the simply supported beam as shown in fig.3 |  |
| c) | A hollow circular column having external diameter 300mm and thickness 15mm carries a vertical load of 150KN at the outer edge of the column. Calculate maximum and minimum intensities of stress in the section. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the moment of inertia about X-X and Y-Y centroidal axis for a tee section shown in fig.4. |  |
| b) | A cantilever of 2.5m length has hollow circular cross section having inner diameter 60mm and outer diameter 70mm. Concentrated load acting at the free end of this cantilever develops maximum bending stress of 75Mpa. Determine the magnitude of this concentrated load. |  |
| c) | A rectangular beam 300mm deep is simply supported over a span of 4m. The moment of inertia of the rectangular beam is 9 x 106 mm4. Determine UDL the beam will carry if the bending stress in the beam is not to exceed 120 MPa? |  |
|  |  | ……3/- |
|  | -3- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft transmits 200KW at 150 r.p.m. If shear stress is not to exceed 70 MPa, determine the diameter of the shaft. For transmission of same power if solid shaft is replaced by hollow shaft having inner diameter 0.6 times outer diameter, determine diameters of hollow shaft and percentage saving in weight compared to solid shaft. |  |
| b) | A shaft transmits 100KW at 200 r.p.m. If the maximum shear stress induced in the shaft is 50 MPa, determine i) Length of shaft ii) Diameter of shaft Take  = 10/m, G= 8 x 104 MPa |  |
| c) | A pipe of external diameter 60mm and 8mm thick is 4m long. It is supported at its both ends. It carries a point load of 200N at its center. Calculate the maximum bending stress induced |  |