## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg.

Subject: Circuit Theory(EE301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Determine the power delivered by current source for the circuit shown in fig.1. |  |
| b) | State and explain maximum power transfer theorem. |  |
| c) | In the circuit of fig.2, applied voltage is given as (0+j10) volts and the current is (0.8+j0.6) amps. Find the value of R and X and also indicate whether X is inductive or capacitive. |  |
| d) | With the help of a power triangle, explain the various types of powers in an a.c. R-L circuit. |  |
| e) | Explain the positives, negative and zero phase sequence component. |  |
| f) | Draw the circuit diagram of De Santy’s Bridge. Give its equation at balance. Mention one advantage & one disadvantage of this bridge. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain Superposition theorem. Find the current through 1 ohm resistance using superposition theorem for the circuit shown in fig.3. |  |
| b) | Find the load current I of the circuit of fig.4 by using source transformation. |  |
| c) | Apply Mesh method to find the loop currents I1, I2 and I3 shown in fig.5. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A circuit consist of 100 resistance, inductance of 0.318H and unknown capacitor in series. When connected across 230V, 50Hz a.c. supply, the current is found to be equal to 23 0. Calculate i) Value of capacitance ii) Voltage across inductance and iii) Power consumed. |  |
| b) | A capacitance of 50F is connected in parallel with a choke having a resistance of 20 and inductance of 0.05H across a 230V, 50Hz supply. Calculate the current drawn by the circuit and power consumed. |  |
| c) | A choke takes a current of 2A when connected across 230V, 1-phase, 50Hz A.C supply and consume 40W. Calculate i) Power factor ii) Inductance of the coil and iii) Resistance of the coil. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An inductor having a resistance of 25 ohms and a Q0 of 10 at a resonance frequency of 10 KHz is fed from a 1000 volts supply. Calculate i) Value of series capacitance required to produce resonance with the coil ii) The inductance of the coil iii) Voltage across capacitance & iv) Voltage across the coil. |  |
| b) | Draw and explain the graphic representation of parallel resonant circuit. | ……..2/- |
|  | -2- |  |
| c) | A coil of resistance 20 ohms and inductance 200 mH is in parallel with a variable capacitor. This combination is in series with a resistor of 8000 ohms. The voltage of the supply is 200V at a frequency of 106Hz. Calculate i) The value of capacitor to gives resonance ii) The Q0 of the coil & iii) The current in each branch of the circuit at resonance. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a phasor diagram, derive the relation between the line voltages and phase voltages in star connection. |  |
| b) | Three star connected inductors take 8 KW at a p.f. 0.8 when connected across a 400V, 3-phase 3-wire supply. Find the circuit constants of the load per phase. What is the phasor sum of the three line currents? |  |
| c) | A balanced delta connected load of (8+j6) ohms per phase is connected to a 3-phase, 400V supply. Find the line current, p.f., power and total reactive power. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The line currents in the R, Y and B lines are IR=100 300 A, Iy=200 240A and IB=500 60A. Calculate the positive, negative and zero phase sequence components. |  |
| b) | Draw Maxwell’s bridge and Wien’s bridge. Give their equation at balance and list at least one advantage and one disadvantage of each. |  |
| c) | Explain what do you mean by transients. Explain the different types of transients. |  |