## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC

Subject: Industrial Electronics(5134)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the symbol of LASCR and explain its working in brief. |  |
| b) | State the wavelengths of ultrasonic waves when travelling through solid, liquid and gas. |  |
| c) | State any six advantages of high frequency heating over conventional type heating. |  |
| d) | State the three important precautions to be taken during maintenance of battery. |  |
| e) | State the different ignition ratings and define each. |  |
| f) | Give three points of distinction between ON line UPS and OFF line UPS. |  |
| g) | State the merits and demerit of Follow up tube circuit with respect to normal heat control circuit, as used for resistance welding. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With the help of neat circuit diagram explain in brief the photo relay, operating with light using transistor and SCR. |  |
| b) | What is the function of single phase preventer? With the help of neat circuit diagram explain the operation of single phase preventer circuit in brief. |  |
| c) | With the help of neat circuit diagram explain in brief the working of water level controller circuit. |  |
| Q.No.3. | Answer any three of the following: | 3 x 4 = 12 |
| a) | With the help of neat circuit diagram, explain in brief the operation of emergency lighting circuit. |  |
| b) | Explain in brief power flasher circuit with the help of neat circuit diagram. |  |
| c) | Explain in brief, the operation of thermistor controlled heater with the help of neat circuit diagram. |  |
| d) | State the applications of ultrasonic vibrations and explain each in brief. |  |
| e) | How can flaws in metals and concrete slabs be detected using ultrasonic vibrations? Hence explain the method with the help of neat block diagram. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain in brief the working principle of Induction heating with the help of neat circuit diagram. Also state its applications. |  |
| b) | i) State the expression for dielectric power loss per unit volume of material in dielectric heating indicating each term used in expression. |  |
|  | ii) State the methods of power coupling and explain each in brief. |  |
| c) | With the help of neat circuit diagram, explain the working of induction generator. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Derive the following equation Tdc + dc = 1 where Tdc is coefficient of torque and dc is coefficient of speed. |  |
| b) | State the methods used for speed control of three phase ac motor and explain each in brief. |  |
| c) | Draw the following characteristics for dc shunt motor: i) Torque v/s armature current ii) Speed v/s Torque. Explain in brief the above characteristics. |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | i) Define the following: i) Percent duty ii) Average current. |  |
|  | ii) The averaging time of size B ignition tube is 7.5 seconds. The working voltage is 460/500 volts. Tube is ON for 5 seconds and OFF for 45 seconds. Calculate percent duty of the tube. |  |
| b) | Draw the Sequential Weld Timer circuit and explain its working in brief. |  |
| c) | Draw the circuit diagram of leading tube trailing tube circuit and explain its working in brief. |  |
| d) | Draw the circuit diagram of capacitive energy storage system used for resistance welding and explain its working in brief. |  |

***********