## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Estimation & Costing(4529)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the advantage of preparing an approximate estimate before a detailed estimate? Give three points. |  |
| b) | Explain item–wise estimate. |  |
| c) | An Abstract sheet is a format containing columns. List the titles of the six columns on it. |  |
| d) | Mention two purposes of Rate Analysis. |  |
| e) | Explain ‘Billing’. |  |
| f) | Explain ‘Estimating’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write out a typical item wise estimate of a one room load bearing structure having R.C.C. roof. Assume all necessary quantities for various items. Write out atleast 6 items. |  |
| b) | What are some of the reasons for preparing an approximate estimate? Explain any 6. |  |
| c) | On the total cost of all items of works in an estimate, a percentage of it is added for two components i) Contingencies ii) service charges for providing water, electricity consumption and sanitary arrangements. Explain these two components. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write out a detailed sanitary estimate for a residential house. Consider any 6 items of works. |  |
| b) | Write the description of any one item of work under each of the following types: (i) R.C.C. (ii) Masonry (iii) Roof (iv) Timber works (v) Metal works (vi) Flooring. |  |
| c) | Explain the basis of deciding the units of measurement for various items of works as (i) m3 (ii) m2 (iii) Running meter (iv) NO (v) Lumpsum (vi) day work. |  |
| Q.No.4. | Calculate the quantities of any three of the following items of works with reference to the drawing given in fig.1. | 3 x 4 = 12 |
|  | i) External Plastering |  |
|  | ii) Excavation for foundation |  |
|  | iii) Tiling of toilet for floor and walls |  |
|  | iv) R.C.C. in lintels |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Explain in detail the main two factors affecting the Rate analysis of an item of work. | (4) |
| b) | Work out the rate analysis of any two of the following items of works: i) RCC for beams – 1:2:4 ii) Fixing of wash basin iii) Provision of 5 amps plug point 3 metres away from the switch board. | 2 x 4 = 8 |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is carpet area estimate? |  |
| b) | What is an item of work? |  |
| c) | Differentiate Measurement sheet and Abstract sheet. |  |
| d) | Explain (i) Contractor’s profit in Rate Analysis (ii) Labour Rate. |  |