## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Welding in Shipconstruction-II(4337) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the importance of pre-heat treatment in welding? Give suitable example from ship construction aspect. |  |
| b) | In ship construction welding where and why is x-ray welding test recommended? |  |
| c) | Give the classification society requirement for welding. |  |
| d) | Summarize the various costs to estimate the total welding cost. |  |
| e) | Briefly explain the welding technique to weld Gray cast Iron. |  |
| f) | What is hard facing? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Describe the MIG welding process as used in ship construction. |  |
| b) | What is the advantage of Submerged Arc welding? Describe the same with a neat sketch. |  |
| c) | With neat sketch explain how jigs and fixtures are used during the welding process. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Give example of dissimilar metals welded in ship construction and explain the concept and the problems faced in welding. |  |
| b) | Detailed out the welding requirement for stainless steel welding. Also describe the welding technique. |  |
| c) | What is the importance of destructive tests in welding? Explain the tensile test of weld. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With suitable examples and sketch, describe various joints in ship construction. |  |
| b) | Calculate the total cost of manual flux shielded metal arc welding using the data given below: Labour and overhead charges per hr = Rs. 250 Electrode consumption = 0.35 Kg/m Electrode price = Rs. 80 per Kg Arc voltage = 22 Volts Arc current = 200 Amps E = 0.6 Rate per KWhr = Rs. 9.8 Weld length = 2.5 metres Welding speed = 12 m/hr Operating factor = 30% |  |
| c) | Describe the use of slot welding in ship construction. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe the radiographic tests of weldments as conducted in ship construction yard. |  |
| b) | Explain cladding of shafts. |  |
| c) | Summarize the safe welding practices. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Health of welders |  |
| b) | Reconditioning machine parts |  |
| c) | Welding symbols |  |
| d) | Post heat treatments |  |

***********