## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the functions of blow off cock. |  |
| b) | What do you understand by the term wet steam? |  |
| c) | Define Brake thermal efficiency. |  |
| d) | State uses of compressed air. |  |
| e) | Define thermal conductivity of a material. |  |
| f) | What is meant by ‘vacuum efficiency’ of a condensing plant? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of water level indicator. |  |
| b) | What is the function of a Super heater? Explain any Super heater with the help of a neat sketch. |  |
| c) | State the advantages of an artificial draught system over natural draught system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the enthalpy of 2Kg super heated steam at a pressure of 52 bar and a temperature of 350C. Assume Cp for super heated steam as 2.2 KJ/KgK. |  |
| b) | Determine the volume of 1Kg of wet steam having dryness coefficient of 0.9 and at a pressure of 15 bar. The volume of water may be neglected. |  |
| c) | Draw a neat sketch of any steam turbine and explain its working. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain Diesel cycle. |  |
| b) | Explain with the help of a sketch, the working of a four stroke I.C. Engine. |  |
| c) | When is multi-stage compression used for air? What are its advantages? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A furnace wall is made up of refractory bricks of 300mm thick. The inner and outer surfaces of the wall have temperature of 1000C and 150C. Find the heat loss per square meter per sec. Take Thermal conductivity as 4.5W/mK. |  |
| b) | With the help of a neat sketch explain the construction and working of a Jet condenser. |  |
| c) | What is the function of cooling tower? Explain any cooling tower with the help of a neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain vapour compression refrigeration system with the help of a neat sketch. |  |
| b) | State the properties of a good refrigerant. What are the normal refrigerants used? |  |
| c) | Write short notes on the following: i) Efficiency of a steam turbine ii) Stefan Boltzman law. |  |

***********