## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Dimensions in the figures are in mm, unless specified.

5) Refer to data provided at the end of the Question Paper.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | In process piping, explain the steps followed to arrive at piping Isometric drawings. |  |
| b) | Give the functions of a Union in a screwed pipeline. Differentiate between a Union and a Coupling. |  |
| c) | Sketch any two pipe supports. |  |
| d) | Give the symbols of the following pipe fittings in flanged, welded and screwed connections in single line: i) Elbow–turned down ii) Tee- outlet up iii) Globe Valve. |  |
| e) | List the different advantages of plastics as a piping material. |  |
| f) | What are corrosion tables? What is its importance? |  |
| g) | Explain the different reasons for using Pipe Bends in a pipeline. |  |
| Q.No.2. | Design a pipe of material ASTM A106 GB to fabricate a pipeline to conduct a fluid at 150C, delivery pressure of 9.2Kg/cm2 and discharge of 100 litres/min. The total length of pipeline is 775mts and in its length are present 7 numbers of 90 short radius Elbows, 2 numbers of Gate valves, 3 numbers of Tees, 2 numbers of Unions and 1 Safety Valve. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Front View, Top View, Side View and Developed View of the portion of the pipeline whose Isometric View is shown in Fig.1. (Draw out of scale but dimension clearly). | (08) |
| b) | Make a Bill of Materials of the pipeline shown in Fig.1, Q.3a. Neglect the weight of pipes and fittings. | (04) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the major phases in piping installation. |  |
| b) | Explain the specifications of the following: i) Pipe ii) Globe Valve iii) Flange. |  |
| c) | List any five piping standards used in Industries. Explain the importance of piping standards. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the Draw Pipe bending processes. |  |
| b) | Draw the end profile of a pipe to fabricate a 4-piece Pipe Bend of 60mm O.D and bend angle 85. |  |
| c) | Explain gas purging of pipes during welding. Explain with neat sketches the different methods and procedure of gas purging. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure to apply Insulation on a pipeline. |  |
| b) | Explain hydro testing of a pipeline. |  |
| c) | Explain the different special design consideration to be given importance while fabrication a steam pipeline. |  |

### DATA

Table 1: - Pressure Drop in Pipes.

| Pipe US NB GPM | Pressure Drop (Psi) per 100 ft long Sch 40 pipe. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1” | 1¼” | 1½” | 2” | 2½” |
| 15 | 8.46 | 2.14 | 0.992 | 0.285 | - |
| 20 | 14.42 | 3.66 | 1.69 | 0.486 | - |
| 25 | - | 5.54 | 2.54 | 0.736 | - |
| 30 | - | 7.79 | 3.60 | 1.030 | 0.424 |

Table 2: - Allowable stresses for ASTM A106 GB.

| Temperature of Fluid F | -20 to 650 | 700 | 750 | 775 |
| --- | --- | --- | --- | --- |
| Allowable Stresses (Psi) | 15,000 | 14,300 | 12,900 | 11,850 |

…….3/-

-3-

Table 3: - Pipe Properties.

| Wall Pipe Thickness NB (inch) (inch) | 40Sch | 80Sch | 160Sch |
| --- | --- | --- | --- |
| 1 | 0.133 | 0.179 | 0.250 |
| 1¼ | 0.140 | 0.191 | 0.261 |
| 1½ | 0.145 | 0.200 | 0.281 |
| 2 | 0.171 | 0.218 | 0.311 |
| 2½ | 0.203 | 0.276 | 0.375 |