## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Building Construction-V(4532)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer following in brief: | 5 x 3 = 15 |  |
| --- | --- | --- | --- |
| a) | Under which soil conditions piles are adapted? |  |  |
| b) | State the purpose of curtain wall. |  |  |
| c) | List three materials used for framing in false ceiling. |  |  |
| d) | List three materials used in making shutters in built in furniture. |  |  |
| e) | What is a dumb waiter? |  |  |
| Q.No.2. | Answer the following Questions: |  |  |
| a) | Write a comparative note on precast piles and cast in situ piles. | (6) |  |
|  | ### OR |  |  |
| a) | Draw a key section through a basement in a building. Assume suitable dimensions and draw details at important locations. | (6) |  |
| b) | Draw neat sketch of a combined footing. Do not show reinforcement details. | (3) |  |
| c) | Where would you adapt the following partitions suitably? i) Precast concrete partition ii) Strawboard partition. | (3) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |  |
| a) | A commercial building requires to be provided with curtain walling. Prepare sketches. Accordingly make suitable assumptions. |  |  |
| b) | Differentiate between partition walls and curtain walls. Give three points atleast. |  |  |
| c) | Draw detailed plan and elevation of a timber framed partition with suitable panelling material. |  |  |
| Q.No.4. | Answer any one of the following Questions: |  | (12) |
| a) | A dining hall of restaurant 10m x 10m needs a false ceiling. Prepare schematic drawing for the same with necessary details. |  |  |
| b) | A bedroom measuring 4m x 4m has to be provided with false ceiling. Prepare a sketch layout and show necessary details. |  |  |
| Q.No.5. | Prepare a layout of study table to house computer and ample book storage. Space available is 1.20m x 60cm. Draw necessary joinery details. |  | (12) |
| Q.No.6. | Answer the following Questions: |  |  |
| a) | Elaborate on considerations for effective use of Built in furniture. |  | (6) |
|  | ### OR |  |  |
| a) | Sketch a design and details for a cupboard in a bedroom. Space available is 90cm x 60cm x 2.50m ht. |  | (6) |
| b) | State the considerations to be made in civil works for provision of elevator in a commercial building. |  | (6) |
|  | ### OR |  |  |
| b) | Explain the following terms: (i) Hoistway (ii) Machine room (iii) Buffer. |  | (6) |

***********