## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term shear stress, shear strain and modulus of rigidity. |  |
| b) | Differentiate between Ductile and Brittle materials with examples. |  |
| c) | Define Resilience, Proof Resilience and modulus of Resilience. |  |
| d) | State and explain Perpendicular Axis theorem. |  |
| e) | Sketch SFD and BMD for a cantilever beam carrying UDL throughout. |  |
| f) | In theory of simple bending define Neutral Axis and moment of Resistance with a neat sketch. |  |
| g) | Sketch shear stress distribution for: (i) Triangular section ii) Circular section. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar consisting of three lengths 250mm, 400mm and 150 mm with diameters 40, 20 and 30 respectively is subjected to an axial pull of 80KN. Determine the total elongation and stresses produced in each portion. E= 2 x 105 N/mm2. |  |
| b) | A reinforced concrete column 350 x 350mm in section is provided with 8 steel bars of 20mm diameter. The column carries a load of 500KN axially. Find the stresses in steel and concrete if Es = 2 x 105 N/mm2 and Ec = 0.14 x 105 N/mm2. |  |
| c) | A bar of steel 30 mm in diameter was subjected to a tensile load of 70KN and the measured elongation on a 200mm gauge length was 0.15mm and change in diameter 0.0038mm. Calculate Poisson’s ratio and Modulus of Elasticity. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A weight of 600 N is dropped on a collar connected to the end of vertical bar of 2.4m long and 25mm diameter from a height of 500mm. Determine instantaneous stress, elongation and strain energy stored by bar. Take E = 210 x 103 N/mm2. |  |
| b) | A steel rod is 20m long at a temperature of 24C. Find the free expansion of the rod when the temperature is raised to 74C. Find the stress produced when i) the expansion of rod is prevented ii) when rod is allowed to expand by 5mm. Take E = 2 x 105 N/mm2 and =12 x 10-6/C. |  |
| c) | Explain the behaviour of mild steel under tension test with neat sketches of failure and stress strain curve and define all the important terms. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam of 3m span carries a UDL of 30KN/m for a length of 2m from free end. Also a point load of 20KN at 1m from fixed end. Draw SFD and BMD of beam. |  |
| b) | A simply supported beam of 4m span carries point loads of 40KN and 60KN at 2m and 3m from left support. Draw SFD and BMD of beam. |  |
| c) | A beam ABC of length 8m is with simple supports at A and B for a span of 6m while BC is overhang for 2m. A UDL of 30 KN/m acts on AB and a point load of 20KN at end C. Sketch SFD and BMD and find maximum +ve and –ve BM in the beam. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the moment of inertia of I section with top flange 120 x 12mm, web 10 x 258mm and bottom flange 240 x 30mm about its horizontal centroidal axis xx. |  |
| b) | A beam of rectangular section supports a UDL of 25KN/m throughout the span of 4 m along with a concentrated load of 20KN at mid span. If the depth of beam is twice the width and maximum bending stress is 75N/mm2, find the size of beam. |  |
| c) | Find the safe concentrated load that can be applied at the free end of cantilever beam of 3m span. Section of beam is hollow square section of 80 x 80mm outer and 70 x 70mm inner. Safe bending stress of material is 60 N/mm2. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | A steel beam of I section with flanges 140 x 12mm (top and bottom) and web 10 x 346 mm. It is subjected to maximum shear force of 160KN. Determine maximum shear stress at neutral axis and sketch shear stress distribution. ### OR | (9) |
| b) | A beam of Tee section with top flange 120 x 12mm and web 10 x 148mm is simply supported for a span of 4.5m with a point load of 200 KN at mid span. Determine the maximum shear stress and sketch shear stress distribution. | (9) |
| c) | A mild steel strap of 100mm wide and 20mm thick is bent into a circular arc of radius 50 metres. Find maximum intensity of stress induced in the strap. Take E = 200 KN/mm2. | (3) |

***********