## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(CE301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List classification of survey based on object of survey. |  |
| b) | Sketch conventional sign to show: i) Cutting ii) Marshy land iii) Railway on survey map. |  |
| c) | Explain terms: i) Datum ii) RL iii) BM. |  |
| d) | Convert bearing: i) 343 20 ii) N 20 30 W iii) 90. |  |
| e) | Explain orientation by back sighting. |  |
| f) | Explain telescopic type of levelling staff. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How will you continue chaining past an obstacle like building? |  |
| b) | Explain procedure for reciprocal ranging with proper sketch. |  |
| c) | What are offsets? Explain procedure to calculate area of cross staff survey with an example, proper tabulation & correct sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is (i) Fore bearing (ii) Back bearing? If FB of line AB is 230, what is bearing of BA? Convert bearings of AB & BA into quadrantal bearing system. |  |
| b) | Explain WCB system with example. Also state how the system is different from quadrantal system. |  |
| c) | Following bearing are taken in running compass traverse. Line FB BB A B 12430 30430 B C 6815 246 C D 31030 13515 D A 200 15 17 45 At what stations do you suspect local attraction? Find corrected bearings. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Describe Rise and fall method of computing levels. Also sketch level page for recording reading. |  |
| b) | Following staff readings were observed successively with level instruments having been moved forward after second, fourth and eighth reading. 0.875, 1.235, 2.310, 1.385, 2.930, 3.125, 4.125, 0.12, 1.875, 2.030 First reading was taken on staff held on BM of elevation 132.135. Enter reading in level book form and reduce the levels. Apply necessary checks. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe briefly temporary adjustment of dumpy level. |  |
| b) | What is levelling? Explain simple and differential level with proper example & sketch. |  |
| c) | Explain with a neat sketch intersection method of plane table survey. | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on: (any three) | 3 x 4 = 12 |
| a) | Classification of survey |  |
| b) | Local attraction |  |
| c) | Profile levelling |  |
| d) | Quadrantal bearing system |  |

***********