## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)/ Theory of Structures(AE405) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the characteristics of a force? |  |
| b) | Explain concurrent and non-concurrent force system. |  |
| c) | Define moment and couple. State their units. |  |
| d) | Show position of centroid of rectangle, triangle, semi-circle. |  |
| e) | What are the materials used in concrete? Explain each. |  |
| f) | What is concrete mix-design? |  |
| g) | What are the properties of reinforcement steel in R.C.C. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using law of parallelogram, find the resultant in magnitude and direction of two forces i) 500 N acting 40 North of East and ii) 800 N acting 30 North of West. |  |
| b) | Find the resultant in magnitude and direction of the following concurrent force system: - 200 N acting at 60 North of East - 300 N acting 30 North of West - 400 N acting due South - 500 N acting 20 South of East |  |
| c) | Find the resultant in magnitude and direction of the following concurrent force system: - 3 KN acting at 70 - 4 KN acting at 180 - 5 KN acting at 220 - 6 KN acting at 340 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw S.F.D. and B.M.D. for cantilever of span 4m fixed at A and free at B subjected to point loads 2000 N and 3000 N at every 2m from fixed end. (Ref. Fig.1) |  |
| b) | Simply supported beam of 6m carries uniformly distributed load of 20 KN/m over entire span. Draw SFD and BMD. (Fig. 2) |  |
| c) | Compare SFD and BMD for a cantilever of span ‘L’ m loaded with (i) point load W at free end (ii) u.d.l. of W N/m over entire span. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the centroid of the section shown in fig.3. |  |
| b) | Explain the load bearing structure with respect to structural design. |  |
| c) | Explain seismic effects on buildings and special considerations in earthquake zones. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify cracks as structural and non–structural cracks and state their causes. |  |
| b) | What are shrinkage cracks? What are the factors affecting the shrinkage cracks? | ….2/- |
|  | -2- |  |
| c) | List out the causes of occurrence of cracks in buildings. Explain cracks due to moisture change in detail. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Concept of limit state method of design |  |
| b) | Corbelling in masonry structures |  |
| c) | Arches and domes |  |
| d) | One way and Two way slabs |  |
| e) | Rigid frame type structure |  |