## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain ‘Transmission efficiency and Gradeability’. |  |
| b) | What are the functions of a spring and damper in a vehicle suspension system? |  |
| c) | What is drawbar pull? |  |
| d) | Distinguish between a sedan and station wagon car body styles. |  |
| e) | What is hot dip galvanization? |  |
| f) | Why is a band brake used in an Automatic gearbox. |  |
| g) | Explain the necessity of ‘Caster Angle’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A motor car which weight 18000N including the 4 road wheels, each of which has an effective diameter of 0.6m, a radius of gyration of 30cm and a weight of 300N, has an engine developing 70KW at 2200 r.p.m. The parts which rotate at engine speed weighs 1100N, with a radius of gyration of 11 cm. The transmission efficiency is 90% and the total road and air resistance at this engine speed in top gear of 0.84:1 is 920N on level. Axle ratio: 6:1. Calculate: i) the acceleration in m/s2 ii) Under these conditions and assuming the acceleration to be uniform, calculate the time required to increase the speed by 40 Km/hr. |  |
| b) | Explain in details Rolling, Pitching & Bouncing. |  |
| c) | Explain in depth the types of resistances influencing the performance of a vehicle. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a Recirculating ball steering gearbox. |  |
| b) | With the help of a neat sketch, explain the working of a hydraulic rack and pinion type power steering. |  |
| c) | Explain various front and geometry parameters for a 4-wheeled vehicle. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a simple sketch, explain the working of an Automatic transmission. |  |
| b) | With the help of a neat sketch, explain the working of a simple planetary gear set. |  |
| c) | With the help of a neat sketch, explain the construction of a Torque convertor. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw and explain the construction of a short-long arm type suspension. |  |
| b) | With the help of a neat diagram, explain the construction and working of a Macpherson strut suspension. |  |
| c) | With the help of a neat sketch, explain the construction and working of a Telescopic shock absorber. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various rust inhibiting techniques to prevent corrosion in Automobiles. |  |
| b) | Distinguish between a Unitary and Framed structure of an Automobile. |  |
| c) | Draw a detailed sketch of a driver seat reclining mechanism. List down all the main parts. |  |

***********