## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enlist attributes of ideal quantity surveyor. |  |
| b) | What are contingencies and work charged establishment? |  |
| c) | What is schedule of Rates? Explain in brief its contents & use. |  |
| d) | State factors considered for carrying out rate analysis. |  |
| e) | State importance of valuation in construction industry |  |
| f) | Explain methods of executing work in PWD organisation. |  |
| g) | What is the purpose of Tender bid? |  |
| h) | Explain briefly various types of specifications. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | A plan and wall section of a single storeyed structure is shown in Fig.No.1. Work out the quantities of any five of the following items: i) Laterite stone masonry in foundation and plinth in cm 1:6 |  |
|  | ii)Laterite stone masonry in superstructure 23 cm thick in CM: 1:6 iii)R.C.C. slab 1:1 ½ :3, 10 cm thick iv)Internal plaster 1:4 cm, 12 mm thick v) Ceramic tile flooring vi) Dadoing for Bath & WC upto 2.0 m height vii)Painting in grillwork for windows & ventilators. |  |
| b) | Prepare an estimate for the portion of the road between chainages 1 to 10 for the following data: |  |
| Chainage; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. RL of Ground (m); 155.3; 154; 153.5; 153.9; 154.1; 154.3; 154.8; 155; 155.2; 155.3. |
|  | - Formation width of road is 15 m - Side slopes 2:1 in cutting & banking - Length of one chain = 30 m - Use market rates for cutting and embankment earthwork - Turfing with grass for embankment sides can be taken as Rs. 40.00 per square meter - Assume uniform falling gradient of 1 in 150 passing through ground level at chainge 1 - Draw cross section and longitudinal also. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Mention different methods of approximate estimate of a building. Explain any one in detail. |  |
| b) | Carry out rate analysis for Manglore file roofing over 10m2 area with wooden batten 45 mm x 25 mm @ 35 cm c/c. |  |
| c) | Carryout rate analysis for 12 mm thick cement plaster in CM 1:6 for 100 m2 quantity of work. |  |
| d) | Define freehold and leasehold property. Briefly explain the factors affecting value of an asset. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Prepare a tender notice for construction of Primary Health Center in your town/village costing Rs. 80 lakhs to be completed in one year and executed under PWD. |  |
| b) | Explain stages in preparation of project and execution of major work as undertaken by PWD. |  |
| c) | Write short notes on Standard Measurement Book and Nominal Muster Roll. |  |
| d) | Explain any five important components of contract document. |  |
| Q.No.5. | Write short notes on any five: | 5 x 3 = 15 |
| a) | Annual Maintenance and Repair Estimate |  |
| b) | Detailed Estimate |  |
| c) | Salvage value v/s Scrap value |  |
| d) | Percentage Rate Contract |  |
| e) | Essentials of a valid contract |  |
| f) | Legal aspects of specification |  |
| g) | Types of tender |  |