## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-II(2006) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define ‘truss’. Also state assumptions made in case of plane truss. |  |
| b) | Write equations for time of flight horizontal range and maximum height in motion of projectile. |  |
| c) | Show graphical representation of work done i) when force is constant ii) when force is variable. |  |
| d) | State D’Alembert’s principle & define momentum. |  |
| e) | Write velocity ratio for i) Differential wheel & axle ii) Worm & Worm wheel iii) Single purchase crab. |  |
| f) | Define ‘Superelevation’ for railway or roadway on curved path. Write expression for angle of banking for vehicle negotiating curved path. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of simple wheel & axel. Also explain working principle and state formula for velocity ratio. |  |
| b) | A double purchase crab has spur wheel having 45 & 50 teeth while pinion have 15 & 20 teeth. Effort handle is 450mm long and effective diameter of load drum is 180mm. If efficiency is 60% while carrying load of 500N, calculate effort required. |  |
| c) | A pile hammer of weight 3000N falls from height of 3.5m on top of pile. Find impulsive force of blow if hammer comes to rest in 1/100th second. Take g = 9.81 m/s2. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A spring is stretched through 50mm. If force required to stretch spring increases at rate of 20N/10mm, determine work done i) When 50mm of spring is stretched ii) When last 20mm is stretched. |  |
| b) | Water tank of capacity 15000 litres is to be filled up in 20 minutes by pump and water is to be lifted through height of 12m. If efficiency of pump is 70%, find rated power of pump. |  |
| c) | A body of mass 4 Kg moving with a velocity of 12 m/s overtakes a body of mass 3Kg moving with a velocity of 5 m/s in the same direction along a straight line and they coalesce to form one body. Find the velocity with which single body (after coalesce) will move. If second body moves in opposite to that of first, find velocity of single body. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flywheel starting from rest rotates at an uniform angular acceleration and after having rotated 60 revolutions its angular velocity is 60 rpm. Find angular acceleration and interval of time to rotate through 50 revolutions. | ……2/- |
|  | -2- |  |
| b) | A wheel 2m in diameter accelerates from rest to 2000 rpm in 10 seconds. Find angular acceleration, number of revolution made by wheel in 10 seconds. Also find linear velocity at point on rim and tangential acceleration of point on rim. |  |
| c) | A body is projected at such an angle that horizontal range is 3 times greatest height attained by body. Find angle of projection, if with this angle range is 360m. Find necessary velocity of projection and time of flight. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What angle of bank, pilot of an aeroplane should maintain, if he wants to fly a horizontal circular path with a radius of 1300m at speed of 400Kmph. |  |
| b) | An elevator cage of mine shaft weighing 8 KN when empty is lifted and lowered by means of wire rope. Once man weighing 600N entered it and lowered with uniform acceleration such that when a distance of 187.5m is covered velocity of cage was 25 m/s. Determine tension in the rope and force exerted by man on floor of cage. |  |
| c) | A train has mass of 200t. If frictional resistance amounts to 6N per tonne, what steady pull must locomotive exert in order to increase speed on level road from 30 Kmph to 60 Kmph within a period of 11/2 minute. |  |
| Q.No.6. | Answer any one of the following Questions: |  |
| a) | Determine forces in members of simply supported truss shown in fig.1 using i) Method of joint (all members) ii) Any three members using method of section. | (8) (4) |
| b) | Determine forces in members of truss shown in fig.2 by method of joints. | (12) |