## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(4214) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With help of block diagram, explain building blocks of a measurement system, describing the various elements, their functions. |  |
| b) | Explain classification of transducers as: i) Analog and digital ii) Self generating and power operated. State examples of each. |  |
| c) | Explain bonding techniques used in strain gauges. What is the need for temperature compensation in strain gauges? |  |
| d) | Explain construction, principle and characteristics of Thermistors. |  |
| e) | Explain working principle and details of variable inductance due to change of self-inductance. |  |
| f) | Explain working principle and details of variable capacitance due to change in distance between plates. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Define Instrumentation. What are the objectives of measurement? Explain any two methods of classification of instruments. |  |
| b) | Describe the applications of a measurement system. |  |
| c) | Write notes on: i) Primary and Secondary standards ii) Primary and Secondary calibrations. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the principle and operation of: i) Thermo-electric transducers ii) Piezo-electric transducers. |  |
| b) | Explain the principle and operations of: i) Bourdon Tube ii) Bellows iii) Flapper Nozzle. |  |
| c) | Explain the construction, characteristics and compare various types of strain gauges. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the principle of operation of potentiometer as a transducer. Describe construction, characteristics and compare various types of potentiometers. |  |
| b) | Explain with block diagram, application of resistance strain gauge in measurement of force, showing output waveform in various stages. |  |
| c) | Write notes on: i) IC sensors ii) Compensation techniques. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | How do variable inductive transducers attain variation in inductance? Explain the various modes, with suitable diagrams. |  |
| b) | With help of block diagram, explain the measurement of pressure, using inductive transducer, showing output waveforms at each state. |  |
| c) | Compare single output and differential output arrangement in inductive transducers. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | How do variable capacitive transducers attain variation in capacitance? Explain the various modes using suitable diagrams. |  |
| b) | With help of block diagram, explain measurement of displacement using capacitive transducers, showing output waveforms at each stage. |  |
| c) | With help of block diagram, explain measurement of level of conductive and non-conductive liquid, using capacitive transducers, showing output waveform at each stage. |  |

***********