## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define bandwidth and frequency response of an amplifier. |  |
| b) | What is the need of Power amplifier? |  |
| c) | Explain the construction of Uni-junction transistor. |  |
| d) | Draw a complete hybrid equivalent circuit of a transistor in CE configuration. |  |
| e) | What is feedback? Explain with neat diagram positive feedback and negative feedback. |  |
| f) | Define Beta cutoff frequency and alpha cutoff frequency. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A transistor is operated in CE mode. Given data: RL = 3.3K, RS = 600, hfe = 200, hie = 6.4K, hre = 1.2x10-4, hoe = 5x10-6 mhos. Calculate (1) Current gain (2) Voltage gain (3) Power gain. |  |
| b) | What are the various coupling schemes of two stage amplifiers? Explain with neat sketch the frequency response of R-C coupled amplifier. Mention any two advantages and disadvantages of R-C coupling. |  |
| c) | Draw the CE amplifier circuit and frequency response curve and explain the effect of an emitter–bypass capacitor on low frequency response of the amplifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat circuit diagram working of RC phase shift oscillator. |  |
| b) | Name the four types of negative feedback and draw block diagram of each. Also list 4 merits of negative feedback. |  |
| c) | Solve the following: |  |
| i) | A certain amplifier has an internal gain of 100 and harmonic distortion in the output is 15%. To improve the performance of the amplifier from the point of view of harmonic distortion, negative feedback is introduced in the circuit. This reduces the distortion with a tolerable limit of 5%. Calculate feedback factor. |  |
| ii) | A certain amplifier gives an output of 20V, for an input of 200mV. The amplifier introduces a harmonic distortion of 15%. The tolerable limit of harmonic distortion in the output is 3%. Calculate feedback factor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of transformer coupled amplifier circuit and draw its frequency response. |  |
| b) | Explain CE amplifier’s high frequency response to a square wave. |  |
| c) | Draw the circuit of push-pull amplifier and explain the working. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between Class A, Class B and Class C power amplifier. |  |
| b) | A common source simplifier uses FET having drain resistance = 200K and amplification factor of 10. Given resistance across the drain rd = 100K. Calculate voltage gain and output impedance. Also comment on the low frequency response of FET amplifiers. |  |
| c) | Explain how FET can be used as voltage variable resistor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of Astable multivibrator and explain the working with the help of waveforms. |  |
| b) | Draw a neat circuit diagram of Schmitt trigger and explain how it converts a sine wave signal into a square wave. |  |
| c) | With necessary sketches explain the construction and characteristics of an SCR. |  |

***********