## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./EE/COMP/EC/MED/E&EE/EI

Subject: Digital Electronics(CS309) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the logic diagram of Half Adder and write its Truth Table. |  |
| b) | Construct OR Gate and AND gate using universal NAND Gate. |  |
| c) | Find the output voltage of a 4 to 1 Binary ladder D/A converter for the input combination: i) 1011 ii) 1110. |  |
| d) | Do as directed i) 25(10) = _________(2) ii) 1101.011(2) = _________(10) iii) CA (16) = ______________(2) |  |
| e) | Show how to convert a R-S Flip Flop to D Flip Flop and write the Truth Table of D Flip Flop. |  |
| f) | Draw the diagram of 4 bit synchronous Down Counter. |  |
| g) | Convert 28(10) into BCD and Gray. |  |
| h) | Prove the following: i) A+AB=A+B ii) A(A+B)=A. |  |
| i) | Draw the diagram of Serial IN Parallel OUT Shift register. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | With a neat diagram, state and prove any one De Morgan’s theorem. |  |
| b) | Draw the logic diagram of 2:4 Decoder using logic gates and write the Truth Table. |  |
| c) | Simplify using K Maps Y = m (2,3,4,5,6,7,10,11,13,14,15) |  |
| d) | Write a note on Parallel Binary Adder. |  |
| e) | Draw the diagram of BCD to 7 Segment decoder driver (common cathode type). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a logic diagram, the operation of JK Master Slave Flip Flop. |  |
| b) | Give the circuit of R-S Flip Flop using NAND gates only. Explain the same with the Truth Table. |  |
| c) | Perform the following Binary Addition and Binary Subtraction using 2’s complement method: i) 1011(2) + 1110(2) ii) 1101(2) – 0101(2) iii) 0101(2) – 1101(2). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic diagram of 4 bit Shift register. Explain how data 1101 is shifted serially IN and taken serially OUT. |  |
| b) | Draw the diagram of 4 bit Asynchronous/Ripple counter. Explain its operation with the help of the count sequence. |  |
| c) | Draw the circuit of synchronous decade counter. Show the Truth Table (count sequence) and the Timing diagram w.r.t. the count sequence. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram, explain the working of Successive Approximation Analog to Digital Converter. |  |
| b) | With a neat diagram, explain the working of Binary ladder type D/A converter. |  |
| c) | Write short notes on: i) Ring Counter ii) ASCII code. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | JK Flip Flop |  |
| b) | Multiplexers |  |
| c) | Asynchronous Down Counter |  |
| d) | Encoders |  |
| e) | Digital and Analog signals |  |

***********