## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by Electric drive? Classify electric drives. |  |
| b) | Give three points of distinction between active and passive load torque. |  |
| c) | State the advantages of electrical braking over mechanical braking. |  |
| d) | Sketch heating curve. Also, state the expression for temperature rise in an electric motor, indicating each term used in the expression. |  |
| e) | State the type of motor used for following drives and justify your choice: i) Reciprocating pump ii) Lathes. |  |
| f) | State the factors on which speed of three phase induction motor depends. |  |
| g) | State the special features of textile motors. |  |
| h) | State the types of selector switches. What is their function? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw joint speed v/s torque characteristics of motor driving a fan and explain it in brief. Also state the significance. |  |
| b) | Explain the variation in load torque with angular displacement of shaft, with the help of neat graph. Also at least two examples of drives for which it is applicable. |  |
| c) | A load torque of 6000 N-m is supplied by motor through gears of ratios 1:2:4:8:16. The speed of the motor is 960 r.p.m. Find the load torque referred to motor shaft if the efficiency of each gear is 85%. Also determine the power required by load and power input to motor if motor efficiency is 88%. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Can regenerative braking be directly applied to D.C. series motor? Give your choice and justify it. |  |
| b) | State the type of rheostatic brakings applicable for three phase Induction motor. Explain any one of such type of your choice with the help of neat diagram. |  |
| c) | With the help of neat diagram, explain counter current braking for d.c. Shunt motor in brief. Also draw its speed v/s torque characteristics. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State the methods used to control the speed of 3 phase slip ring Induction motor and explain frequency control method used to control the speed of squirrel cage Induction motor in brief with the help of neat circuit diagram. |  |
| b) | What is the significance of voltage to frequency control in three phase Induction motor? | ……2/- |
|  | -2- |  |
| c) | What is slip power? How can it be recovered? Draw a neat circuit diagram for using slip power recovery control in brief. |  |
| d) | State the effect of injecting an external emf into the rotor circuit on induction motor operation. Explain in brief this effect under different conditions on 3 phase Induction motor, with the help of neat phasor diagrams. |  |
| e) | List out the different classes of duty used for electric motor. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State the expression for determining power rating of motor for following drives: i) Lifts ii) Pump iii) fan, indicating clearly each term used in expression. |  |
| b) | Explain in brief the effect of load inertia on rating of motor, with the help of neat graphs. |  |
| c) | State the drives, which are subjected to load fluctuations. How can such fluctuations be minimized? Explain the method in brief. |  |
| d) | Give two points of distinction between: i) Limit switch and Proximity switch ii) Contractor and Relay. |  |
| e) | State the types of Timers. Explain the working of a Timer in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of interlocking operation? State the types of interlocking control circuits used in the operation of electric motor and explain any one on such type of your choice in brief with the help of neat control circuit diagram. |  |
| b) | Draw a neat control circuit diagram to indicate the operation of electric motor fed from two sources of supply. |  |
| c) | Draw a power circuit diagram and explain the operation of direct reversing of 3 phase induction motor. |  |

***********