## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms: (i) Feeder (ii) Distributor (iii) Service mains. |  |
| b) | Define the terms: (i) Connected load (ii) Demand factor (iii) Diversity factor. |  |
| c) | State any six important properties that an overhead line insulator must possess. |  |
| d) | With the help of neat diagram, explain the procedure of installing surface conduit wiring system. |  |
| e) | With the help of neat diagram, explain in brief the working of MCB. |  |
| f) | List any six methods to improve earth resistance. |  |
| g) | Explain in brief ABC system for rural Electrification. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive & compare the volume of conductor material required in DC 2 wire system with one conductor earthed with 3 phase 4 wire AC system. |  |
| b) | Explain any one method of solving A.C. distribution calculations. Also draw phasor diagram of AC Distributor. |  |
| c) | A single phase AC Distributor AB 300 mtrs long is fed from end A and is loaded as under: - 100 A at 0.707 p.f. lagging 200 mtr from point A - 200 A at 0.8 p.f. lagging 300 mtr from point A. The load resistance and reactance of the distributor is 0.2 Ω and 0.1 Ω per kilometer. Calculate the total voltage drop in the distributor. The load power factors refer to the voltage at the far end. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of guy assembly. Label its parts and state their functions. |  |
| b) | What is the role of insulator in distribution system? State any five advantages of suspension insulator over pin type insulator. |  |
| c) | Estimate the main material requirement for a 750 mtr, 415/240 V three phase line with four wires in vertical configuration. The lines emanate from a substation to feed a load of 30 KW. Take the span between two poles as 50 metres. The size of the conductor is ACSR 6/1 x 2.59 mm (code weasel). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain SWER wire system with its advantages and disadvantages. |  |
| b) | Briefly explain the below mentioned faults & their effect on distribution system: (i) Lightning (ii) L-L fault. |  |
| c) | Explain ‘Staircase wiring’ & ‘Godown wiring’. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any six IE Rules relevant to Domestic wiring system. |  |
| b) | State any six important considerations regarding motor installation wiring. |  |
| c) | In a workshop, one 15HP, 400 volts three phase 50 Hz Induction motor is to be installed. |  |
|  | i) Draw a single line diagram for the motor installation. |  |
|  | ii) Find full load current of the motor. |  |
|  | iii) Find the size of the cable to be used. |  |
|  | iv) Find the rating of the switch fuse. |  |
|  | v) Draw wiring diagram for above installation. Given efficiency of the motor is 0.8 and p.f. is 0.85. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the working of MCCB. State its advantages and disadvantages. |  |
| b) | With the help of neat diagram explain pipe earthing. |  |
| c) | i) What is Earthing? Explain the purpose of Earthing. |  |
|  | ii) List out any three types of switches and their applications. |  |

***********