## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define: i) load factor ii) Diversity factor. |  |
| b) | Why maximum demand of a group of consumers is always less than the sum of individual maximum demand? |  |
| c) | Enlist various types of hydroelectric power plants. Which one is most suited as peak load plant? |  |
| d) | Enlist the types of conductors used for transmission of electrical energy. |  |
| e) | Which parameters of short transmission line is neglected for calculation purpose? Why? |  |
| f) | What do you mean by transposition? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of bare load and peak load power plant with an example. |  |
| b) | State and explain the various devices used for preheating of feed water in a thermal power plant. Also draw the schematic diagram indicating location of these devices. |  |
| c) | Explain in brief the major components of diesel generator set. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the reasons for failure of insulators. |  |
| b) | i) List out the different types of non-conventional sources of energy. ii) Explain tidal power generation in brief. |  |
| c) | A generating station has a maximum demand of 25MW, a load factor of 60%, plant capacity factor of 50% and a plant use factor of 72%. Find i) The reserve capacity of the plant ii) The daily energy produced and iii) Maximum energy that could be produced daily if the plant while running as per schedule, were fully loaded. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the main components of HVDC transmission system and state its advantages over HVAC system. |  |
| b) | Write a short note on effect of variation of Transmission voltage and load p.f. on performance of short transmission lines. |  |
| c) | Explain the formation of corona. Give its advantages. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | An overhead 3 phase transmission line delivers 5000 KW at 22KV at 0.8 p.f. lagging. The resistance and reactance of each conductor is 4Ω and 6Ω respectively. Determine i) Sending end voltage ii) Percentage regulation. |  |
| b) | A 3 phase, 50Hz transmission line 100Km long delivers 20MW at 0.9 p.f. lagging at 110KV. The resistance and reactance of the line per phase per Km are 0.2Ω and 0.4Ω respectively, while capacitive admittance is 2.5 x 10-6 siemens/Km/phase. Calculate the current and voltage at the sending end. Use nominal T method. |  |
| c) | Write a short note on vibration dampers. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the effect of wind and ice deposition on the sag? |  |
| b) | A transmission line has a span of 200m between level supports. The conductor has a cross-sectional area of 1.29cm2, weighs 1170Kg/Km and has a breaking stress of 4218 Kg/cm2. Calculate the sag for a safety factor of 5, allowing a wind pressure of 122 Kg per square meter of projected area. What is the vertical sag? |  |
| c) | Explain in brief the various supporting structures used in overhead transmission line. |  |

***********