## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EI/IS/EC/COMP/MED/E&EE

Subject: Microprocessors-I(8085)[4142] [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give any three functions of Accumulator. |  |
| b) | State the Microprocessor initiated operations. |  |
| c) | Define Subroutine. |  |
| d) | If the clock frequency of a Microprocessor is 5 MHZ, how much time will be required to execute an instruction of 18 T states? |  |
| e) | List the different 8085 Interrupts along with their call locations. |  |
| f) | Data byte 28H is stored in Register B and data byte 97H is stored in accumulator. Show the contents of Register B, Register C and Accumulator after executing the following two instructions: MOV A, B MOV C, A |  |
| g) | Explain the concept of Interfacing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by address lines, data lines and control/synchronisation lines in IC 8085. |  |
| b) | Draw the functional block diagram of IC 8085. |  |
| c) | With a neat diagram, explain the flag register of IC 8085. |  |
| d) | Give the functions of all the peripheral initiated signals of IC 8085. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on addressing modes for the instructions in IC 8085. |  |
| b) | Assuming the machine codes for instruction STA 2065H are stored in memory locations 2010H, 2011H and 2012H, explain how many machine cycles will the instruction take to be executed and what happens during each of them. |  |
| c) | Give the meaning, Number of bytes, Number of machine cycles and Number of T States for each of the following: i) CMP M ii) DAA iii) MVI C, 96H |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to find the largest of 10 numbers stored in memory locations starting at 5000H. Let Register C represent the largest number. |  |
| b) | Write a program to count continuously in Hexadecimal from FFH to 00H in a system with a 0.5 sec clock period. Use Register C to set up a 1 millisecond (1ms) delay between each count and display the numbers between the count. |  |
| c) | 10 data bytes are stored in consecutive memory locations starting from 3000H. Write a program to count the number of even numbers in the given series. Let Register E indicate the count of even numbers. | ……..2/- |
|  | -2- |  |
| d) | Write a program to: i) Clear the accumulator ii) add 47H iii) subtract 92H iv) add 64H v) display the results after subtracting 92H and after adding 64H on the output ports. Specify the answers you would expect at the output ports. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Explain the concept of Multiple Ending Subroutine with an example. | (3) |
| b) | Explain the execution of CALL instruction. | (6) |
| c) | Write a note on PUSH instruction related to stack. | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on RESTART instructions. |  |
| b) | Explain the 8085 Interrupt process in detail. |  |
| c) | What do you understand by the following Interrupts: i) TRAP ii) RST 7.5 iii) RST 5.5. |  |

***********