## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Engineering & Technology

Subject: Applied Physics-I(GN103) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define least count of Vernier Calipers. |  |
| b) | Define uniform velocity. |  |
| c) | What is scalar quantity? Give an example. |  |
| d) | Define bulk modulus. |  |
| e) | What is cohesive force? |  |
| f) | State Charle’s law. |  |
| g) | Give two examples for heat transfer by radiation. |  |
| h) | Define electric intensity. |  |
| i) | State 2 properties of magnetic lines of force. |  |
| j) | Define angle of repose. (Only for Mechanical Engg. & Allied branches) |  |
| j) | State two pentavalent impurities used to add in semiconductor. (Only for Electronics Engg. & Allied branches) |  |
| Q.No.2. | Answer any four of the following: | 4 x 3 = 12 |
| a) | State all the fundamental quantities of SI system. Write their units and symbols. |  |
| b) | Obtain the dimensional formula of the following: (i) Pressure (ii) Energy. |  |
| c) | State three differences between distance & displacement. |  |
| d) | A scooter starts from rest and moves with a constant acceleration of 1.2 m/s2. Determine its velocity, after it has traveled for 60m. |  |
| e) | A body is dropped from the top of a tall building. If it takes 2.8 seconds in falling on the ground, find the height of the building. |  |
| f) | A car travels with a uniform velocity of 20 m/s for 5 seconds. The brakes are then applied and the car is uniformly retarded. It comes to rest in further 8 seconds. Draw a graph of velocity against time. Use the graph to find i) distance travelled in first 5 seconds ii) distance travelled after the brakes are applied iii) acceleration in last 8 seconds. |  |
| Q.No.3. | Answer any four of the following: | 4 x 3 = 12 |
| a) | Draw stress v/s strain graph and explain. Define yield point. |  |
| b) | A load of 4 Kg when suspended from the lower end of a wire of length 2m, diameter 1mm, extends it by 0.5mm. Find value of Young’s Modulus. |  |
| c) | Define surface tension. Explain its application. |  |
| d) | A capillary tube of diameter 2.4 mm is dipped in mercury of density 13.6 x 103 Kg/m3. The mercury inside the capillary is depressed through 6 mm. If angle of contact for mercury and glass is 140, find the surface tension of mercury. |  |
| e) | State Newton’s law of viscosity. Write an expression. |  |
| f) | Explain streamline and turbulent flow. What is critical velocity? | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any four of the following: | 4 x 3 = 12 |
| a) | Explain heat transfer by conduction. Give an example. |  |
| b) | A nickel plate of thickness 3 mm has temperature difference of 30C between its faces. It transmits 200 Kcal per hour through an area of 5 cm2. Calculate the thermal conductivity of nickel. |  |
| c) | Drive general Gas equation. |  |
| d) | To what temperature a gas at 0C must be heated at constant pressure so that its volume is doubled? |  |
| e) | Obtain an expression for effective capacitance when two capacitors are connected in parallel. |  |
| f) | Explain the principle of capacitor. |  |
| Q.No.5. | Answer any four of the following: | 4 x 3 = 12 |
| a) | Calculate the potential at a point 0.4 m from a charge of 22 x 10-9 C placed in medium of dielectric constant 2.25. |  |
| b) | State Coulomb’s law of electrostatics. Write an expression. |  |
| c) | Two capacitors of 0.6 F & 1.4 F are connected in parallel and then charged with 250C. Find (i) capacitance of the combination (ii) charge on each capacitor. |  |
| d) | Explain Oersted’s experiment. Write the conclusion of the experiment. |  |
| e) | Define i) Intensity of magnetic field ii) Magnetic flux density. |  |
| f) | A wire carrying current of 3 ampere is 0.4 m long. If it is kept in a magnetic field at strength 3.6 x 10-3 wb/m2 at an angle of 30 with the field, calculate the force acting on the wire. |  |
|  | #### Only for Mechanical Engg. & Allied branches |  |
| Q.No.6A. | Answer any four of the following: | 4 x 3 = 12 |
| a) | A circular coil has 25 turns each of radius 0.12m and is placed in air. If it carries a current of 1.5 A, find the magnetic induction at the center of the coil. |  |
| b) | State and explain triangle law of forces with neat diagram. |  |
| c) | Two forces 48 N and 30 N are acting at 110 with each other. Find their resultant. |  |
| d) | State laws of friction. |  |
| e) | A force of 1000 N is acting at angle 20 with the X-axis. Find its resolved parts along X & Y axis. |  |
| f) | Explain different systems of forces. |  |
|  | Only for Electronics Engg. & Allied branches |  |
| Q.No.6B. | Answer any four of the following: | 4 x 3 = 12 |
| a) | A circular coil has 25 turns each of radius 0.12m and is placed in air. If it carries a current of 1.5 A, find the magnetic induction at the center of the coil. |  |
| b) | Draw and explain energy band structure for semiconductor. |  |
| c) | Explain the formation of P-type semiconductor. |  |
| d) | What is a pn junction? Explain the formation of potential barrier in a pn junction. |  |
| e) | Which are most commonly used semiconductors and why? |  |
| f) | Define (i) Forbidden energy gap (ii) Covalent band. |  |

***********