## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is Soil Mechanism? Enlist the fields of application of soil mechanics. |  |
| b) | With the help of a neat diagram explain in brief soil phase system. |  |
| c) | State any five ways used to identify the soil in field. |  |
| d) | How does void ratio and unit weight of soil affect the permeability? |  |
| e) | Write any four basic points of difference between compaction and consolidation. |  |
| f) | Write in brief what are drained and undrained shear strength tests on soils? |  |
| g) | Explain any two types of faults in rocks. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With reference to standard rotations, derive: - γ=(G+eSr)γw1+e ii) e=wG |  |
| b) | How are consistency limits helpful to engineers in indicating properties and classification of soil? |  |
| c) | With the help of neat sketch explain any one method of sedimentation analysis of soil. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch, explain how soil is tested for shear strength in triaxial shear test. |  |
| b) | Why plate load test is conducted? Explain its procedure and write down the limitations of plate load test. |  |
| c) | With the help sketch (graph) explain how light and heavy compaction affects the OMC (optimum moisture content) of soil. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A falling head Permeameter accommodates a soil sample 6.5 cm high and 48cm2 in cross sectional area. The permeability of soil is 1x10-4 cm/sec. If the fall of head in standpipes is 48cm to 30cm in 45 minutes, determine the size of the standpipe required. |  |
| b) | Define the terms: i) Seepage pressure ii) Pore pressure iii) Quick condition. |  |
| c) | Give a neat labelled sketch related to coulomb’s theory of earth pressure. Explain the meaning of lateral earth pressure. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is bore hole log? Which properties of soil are determined using geophysical methods of soil testing? |  |
| b) | Explain in detail each of the following terms with respect to rocks: i) Folds ii) Horst and Graben iii) unconformity. | …….2/- |
|  | -2- |  |
| c) | i) Give examples and write down the uses of Igneous, Sedimentary and Metamorphic rocks in civil engineering. | (3) |
|  | ii) Write any two points of difference between porosity and voids ratio. | (3) |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | A-line diagram of soil |  |
| b) | Direct shear test |  |
| c) | Soil sampling |  |
| d) | Core cutter method |  |
| e) | Saturated density and submerged density |  |
| f) | Bearing capacity of soil |  |

***********