## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer in brief: | 10x1.5=15 |
| --- | --- | --- |
| a) | Why no flux is required for welding mild steel? |  |
| b) | Give one example each of neutral oxide and amphoteric oxide. |  |
| c) | State two properties of hydrogen fuel gas. |  |
| d) | State one difference between Welding & Soldering. |  |
| e) | Name any two shielding gases used in welds. |  |
| f) | What is electro-slag welding? |  |
| g) | State one characteristic and one industrial application of Nickel. |  |
| h) | Why is it necessary to use a flux in welding? |  |
| i) | State the formula for calculation of carbon equivalent for alloy steel. |  |
| j) | Why there is need for porous filters in acetylene cylinders? |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain how oxygen is produced by electrolysis of water. |  |
| b) | Explain briefly the two methods used for gasification of liquid oxygen. |  |
| c) | Why there is need for preheating carbon dioxide during welding? |  |
| d) | Write a note on explosibility of acetylene in mixture with other gases. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the effects of the following impurities in acetylene: i) Phosphine ii) acetone vapour. |  |
| b) | State any four advantages of using dissolved acetylene. |  |
| c) | Briefly explain the following terms w.r.t. fuels: i) Ignition temperature ii) Calorific value iii) Flame temperature iv) Flame velocity. |  |
| d) | List the different types of oxy-acetylene flames and state their properties & applications. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | How does heat transfer from flame to metal takes place in oxy-acetylene flame? |  |
| b) | Write on distribution of specific heat flux w.r.t. oxy-acetylene flame. |  |
| c) | Explain in brief the three types of soldering fluxes. |  |
| d) | Why there is need for post cleaning of fluxes? |  |

………2/-

-2-

| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | What are the advantages of using flux coated electrodes? |  |
| b) | What are the functions performed by the following flux ingredients: i) de-oxidising elements ii) alloying elements. |  |
| c) | Why there is need to take utmost care in handling & storage of electrodes? |  |
| d) | What are the effects of the following gases absorbed in weld pool: i) nitrogen ii) water-vapour. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain briefly the following faults caused by gas absorption in welds: i) piping ii) blow holes. |  |
| b) | What are clad metals? Why is cladding done? |  |
| c) | Explain BCC. |  |
| d) | What is the effect of temperature on crystal size in metals? |  |

***********