## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.N.1 is compulsory. Answer any five from the remaining Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Weight distribution curve for ship shaped vessel |  |
| b) | Shearing force at a particular point on vessel |  |
| c) | Tensile and compressive stress |  |
| d) | Girders used in way of shell, deck and bottom |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | With a neat sketch explain the procedure to find the stress in deck and bottom of the vessel when bending moment is known. | (10) |
| b) | How the section modulus of a vessel can be increased by keeping the weight of the vessel to minimum? | (2) |
| Q.No.3. | Sketch and describe a typical shell expansion plan. | (12) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Draw the mid-ship section of a bulk carrier with combined framing system and label all the structural members. | (10) |
| b) | Show details of welding joint between tank top plate and lower hopper. | (2) |
| Q.No.5. | If a vessel having section as shown in fig (01) is subjected to a sagging bending moment of 53,394 t-m, calculate the stress exerted in deck and bottom. State their type. | (12) |
| Q.No.6. | The hull of a box shaped vessel is 50m long and has a mass of 600t. The mass of the hull is uniformly distributed over its length. Machinery of 200t mass extends uniformly over the quarter length mid-ship. Two holds extending over the fore and aft quarter length of each have 140t of cargo slowed uniformly over their length. Draw the curve of shearing force and bending moment for this condition and state their maximum values. | (12) |
| Q.No.7. | Answer the following Questions: |  |
| a) | Write the flexure formula used in bending moment calculation and define all the terms used in the formula with units. | (6) |
| b) | Explain the use of flexure formula in determining the section modulus of a transverse bulkhead stiffener. | (6) |