## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Computer Engg.

Subject: Computer Installation, Maintenance & Diagnosis(4226)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Why is a dust free Environment desirable in a Computer Centre? |  |
| 2) | Differentiate between Linear Power Supply and Switch Mode Power Supply. |  |
| 3) | What is the function of the BIOS? |  |
| 4) | What are Intermittent faults? Give one example. |  |
| 5) | What is a Virus? What are the different types of Viruses? |  |
| 6) | List three beep codes and three error codes and state what they mean. |  |
| Q.No.II. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | What steps will you take for site preparation of a Computer Centre? |  |
| 2) | Explain how will you ensure stable power supply to a Computer Centre. |  |
| 3) | Explain the need for upgrading Computer System. |  |
| Q.No.III. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | What steps will you take while installing and commissioning a new Computer System? |  |
| 2) | What is a Chipset? Explain major features of any two Chipsets that you have worked with. |  |
| 3) | Explain BIOS error messages. |  |
| Q.No.IV. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | What are repair generated failures? |  |
| 2) | Explain how typical components in a Mother Board fail. |  |
| 3) | Explain the role of documentation in computer trouble shooting and repair. |  |
| Q.No.V. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | What is a head crash? How will you prevent it? |  |
| 2) | Explain typical PC display problem and the remedial action? |  |
| 3) | List typical PC start up problem, causes and remedies. |  |
| Q.No.VI. | Write short notes: (Any 3) | 3 x 4 = 12 |
| 1) | Overload protection devices |  |
| 2) | Upgrading a Computer System |  |
| 3) | Internet setup |  |
| 4) | Hard Disk Drive problems |  |
| 5) | Preventive & remedial maintenance |  |
| 6) | POST |  |

***********