## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

#### Subject: Basic Ship Theory-II(4335)/(SH401) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Stowage factor |  |
| b) | Wall sided formula |  |
| c) | Cross curves of stability |  |
| d) | Side launching |  |
| e) | Bale and grain capacity |  |
| f) | Floodable length |  |
| g) | Load curve |  |
| Q.No.2. | A box shaped vessel 64m x 10m x 6m floats in salt water on an even keel at 5m draught. A forward compartment 6m long and 10m wide extends from the outer bottom to a height of 3.5m and is full of cargo of permeability 25%. Find the new draughts if the compartment is now bilged. KG=3.5m | (12) |
| Q.No.3. | Answer the following Question: |  |
| a) | Draw a set of typical end launching curves. | (6) |
| b) | When a ship of 10000 tonnes displacement is heeled 15 degrees, the righting lever is 0.2m, KM=6.8m. Find the KG and the moment of statical stability. | (6) |
| Q.No.4. | A uniform box shaped barge 40m x 12m beam is divided into four cargo compartments 10m long. The barge is loaded with 600t of iron ore and level stowed as follows: No.1 hold–135t, No.2 hold-165t, No.3 hold-165t, No.4 hold-135t. The loaded barge floats on an even keel draught of 1.75m. Construct a curve of shear force and bending moment for this condition. Also find the position and value of maximum bending moment for the still water condition. Assume the load to be of uniform distribution. | (12) |
| Q.No.5. | Answer the following Question: |  |
| a) | Describe the load, shear force and bending moment of ship in still water. Explain how these diagrams are drawn. | (6) |
| b) | ### Explain how section modulus of a ship is calculated. | (6) |
|  | ### OR |  |
| Q.No.5. | The mid ship section of a box shaped vessel is as shown in fig.1. Find the section modulus of the deck and keel if the vessel is subjected to sagging bending moments of 55000 t-m. Calculate the stress exerted in deck and bottom. | (12) |
|  |  | ……2/- |
|  | -2- |  |
|  | (Only for students of Revised scheme) |  |
| Q.No.6. | A ship of 10000 tonnes displacement has righting levers as follows: Heel; 0; 10; 20; 30; 40. GZ(m); 0; 0.09; 0.21; 0.30; 0.33. Calculate the dynamic stability to 40 degrees heel. | (12) |
|  | (Only for students of Old scheme) |  |
| Q.No.6. | Determine the gross tonnage and net tonnage of a vessel having total enclosed volume 4500m3 of which 3600m3 is cargo volume. Total crew on board is 9. The depth of vessel is 4.5 m and draft is 3m. | (12) |