## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Medical Electronics

Subject: Electronic Devices & Circuits(MX401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain why a transistor should be biased. |  |
| b) | What are the requirements of a biasing circuit? |  |
| c) | List any six advantages of Negative feedback. |  |
| d) | Explain conditions under which a feedback amplifier works as an oscillator. |  |
| e) | Draw symbol and characteristics of Triac. |  |
| f) | Draw circuit diagram for single phase full wave controlled rectifier with R-L load and waveform for load voltage. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain the base bias circuit for transistor. |  |
| b) | Explain voltage divider biasing circuit for a transistor in detail. |  |
| c) | Compare different types of transistor amplifier coupling. (give four points) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain single stage CE amplifier. |  |
| b) | Draw block diagram of voltage series feedback amplifier and show that the voltage gain is AF=A1+AB. |  |
| c) | Explain with neat diagram RC-coupled transistor amplifier. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat diagram and explain Hartley oscillator. |  |
| b) | Explain working of Colpitt’s oscillator in detail. |  |
| c) | With the help of a neat diagram explain Tuned collector oscillator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of SCR in forward biased mode with the help of characteristics. |  |
| b) | Explain UJT relaxation oscillator in detail. |  |
| c) | Explain characteristics of DIAC and draw its symbol. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain single phase half wave controlled rectifier with resistive load. |  |
| b) | Explain single phase bridge type inverter. |  |
| c) | Explain working of phase control circuit using DIAC and TRIAC. |  |

***********