## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Generation, Transmission & Distribution(EE304)

[Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Demand factor ii) Load factor. |  |
| b) | Give formula for computing inductance & capacitance for single phase line. |  |
| c) | Briefly explain 3 phase, 4 wire AC system. |  |
| d) | What do you mean by earthing? Why it is necessary? |  |
| e) | List any six methods of improving earth resistance. |  |
| f) | State any two advantages and one disadvantage of concealed conduit type wiring. |  |
| g) | Give 3 points of comparison between AC 2 wire and AC 3 wire system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain principle of generation of electricity in Hydroelectric power station. Explain functions of any two components used in such stations. |  |
| b) | What are the advantages and disadvantages of diesel power station? |  |
| c) | Briefly explain features of power generation using: i) Solar energy ii) Wind energy. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between single circuit tower and double circuit tower. |  |
| b) | State function of insulator. List desirable properties of insulator. Briefly explain pin type insulator. |  |
| c) | Define string efficiency. What is its importance? How it can be improved? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain function of stay insulator. |  |
| b) | State advantages and disadvantages of corona. How corona can be reduced? |  |
| c) | Define sag. Explain its effect on design of transmission system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain ring distributor system. State its advantages and disadvantages. |  |
| b) | Compare DC 2 wire and AC three phase – 3 wire distribution system. |  |
| c) | Explain the concept of feeder, distributor and service mains. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain pipe earthing. |  |
| b) | Explain PVC casing & capping type of wiring. State its advantages and disadvantages. |  |
| c) | A Hall of 10m x 4m x 4.5m is required to be fitted with 8 tube lights and 6 fans. Draw electrical circuit & plan showing suitable location of tube lights and fans |  |

***********