## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the function of splitter, notch and deflector in a Pelton wheel. |  |
| b) | A jet of water of diameter 60mm moving with a velocity of 40m/s, strikes a curved fixed symmetrical plate at the center. Find the force exerted by the jet of water in the direction of the jet, if the jet is deflected through an angle of 120 at the outlet of the curved plate. |  |
| c) | State three advantages of centrifugal pump over reciprocating pump. |  |
| d) | State the functions of air vessels in reciprocating pump. |  |
| e) | Write the symbols for i) 5/2 way direction control valve ii) Quick exhaust valve iii) Non return valve. |  |
| f) | Write three major differences between an impulse turbine and a reaction turbine. |  |
| g) | Name any two types of oil seals and state their applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel has a mean bucket speed of 10m/s with a jet of water flowing at the rate of 0.7m3/s under a head of 30m. The buckets deflect the jet through an angle of 160. Calculate the power and the efficiency of the turbine. Assume co-efficient of velocity as 0.98. |  |
| b) | The inward flow reaction turbine develops 735 KW at 750 r.p.m under a net head of 100m. The guide vanes make an angle of 15 with the tangent at inlet. The axial length of the blade at inlet is 0.1 times the outer diameter. The radial velocity of flow through the wheel is constant and the discharge from the wheel is radial. The blade thickness blocks 5 percent of the area of flow at inlet. The hydraulic efficiency of the wheel is 88% and overall efficiency is 84%. Determine i) The wheel diameter ii) The wheel width and iii) The blade angle at inlet. |  |
| c) | Draw and explain the main characteristic curves of Francis turbine and Kaplan turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Kaplan turbine develops 22000 KW at an average head of 35m. Assuming a speed ratio of 2, flow ratio of 0.6, diameter of the boss equal to 0.35 times the diameter of the runner and an overall efficiency of 88 percent, calculate the diameter, speed and specific speed of the turbine. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | The following are the results of tests conducted on a centrifugal pump. Negative pressure head in suction pipe close to pump = 22cms of mercury Pressure head in delivery pipe close to pump =1.5 bar Differences of level between the two gauges = 0.5 m Shaft power of electric motor = 24 KW Discharge of the pump = 0.1 cumec Diameter of the suction pipe = 20 cms Diameter of the delivery pipe = 18 cms Calculate the overall efficiency of the pump. |  |
| c) | Draw a neat sketch of a centrifugal pump and explain the function of all the parts. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The piston diameter and stroke length of a single acting reciprocating pump are 150mm and 300mm respectively. The center of the pump is 5m above the water level in the sump and 33m below the delivery water level. Both the suction and delivery pipes have the same diameter of 75mm and are 6.5m and 39m long respectively. If the pump is working at 30 r.p.m., determine the pressure head on the piston at the beginning, middle and end of both suction and delivery strokes. Take atmospheric pressure head = 10.3m of water and co-efficient of friction f = 0.01 for both the pipes. |  |
| b) | A single acting reciprocating pump, running at 50 r.p.m. delivers 0.00736 m3/s of water. The diameter of the piston is 200mm and stroke length 300mm. The suction and delivery heads are 3.5m and 11.5m respectively. Determine i) Theoretical discharge ii) Co-efficient of discharge iii) Percentage slip of the pump iv) Power required to run the pump. |  |
| c) | Describe multistage centrifugal pumps with i) Impellers in series ii) Impellers in parallel. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a vacuum pump. |  |
| b) | Using graphic symbols, draw the counterbalance circuit and explain its working. |  |
| c) | Discuss the effect of acceleration and friction in suction and delivery pipes of a reciprocating pump. Explain with the help of indicator diagram. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat labelled sketch of a screw pump and explain its working. |  |
| b) | Using graphic symbols, draw a bleed off circuit and explain its working. |  |
| c) | State the functions of FRL unit in a pneumatic circuit. |  |
| d) | What are the different types of methods adopted for the actuation of direction control valves in pneumatic systems? |  |
| e) | Define specific speed of a turbine. What is its significance in selection of turbines? |  |
| f) | A nozzle of 50mm diameter delivers a stream of water at 20m/s perpendicular to a plate that moves away from the jet at 5 m/s. Find the work done by the jet on the plate and efficiency of the jet. |  |

***********