## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg.

Subject: Utilisation of Electrical Energy(EE403) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Briefly explain how heat transfer takes place by radiation? State formula used to compute heat transfer by radiation. |  |
| b) | Briefly explain principle of arc welding. How it is different from resistance welding? |  |
| c) | State any three advantages of resistance heating. |  |
| d) | Draw Speed-Time curve for sub-urban and urban train. |  |
| e) | State the dc motor preferred for traction. Give any 2 reasons for your choice. |  |
| f) | What is submerged arc furnace? State its application. |  |
| g) | Define i) MHCP ii) Lumen. |  |
| h) | State any three advantages of electrical heating. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain any two methods of temperature control in resistance heating. |  |
| b) | State and explain the desirable properties a heating element should posses. State any two heating element materials. |  |
| c) | A resistance oven employing nichrome wire is to be operated from 220V, Single-phase supply and is to be rated at 16KW. If average temperature of charge is 500C and temperature of element is to be limited to 1170C, find diameter and length of electrical wire. Given: Radiating efficiency = 0.57, Emissivity = 0.9, Sp. Resistance of nichrome = 1.9 x 10-6 m. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain core type induction furnace. State its applications. (any one type) |  |
| b) | With neat diagram explain construction and working of core less, high frequency induction furnace. |  |
| c) | With neat diagram explain conducting bottom and non-conducting bottom arc furnaces. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare AC welding with DC welding. (6 points) |  |
| b) | State and explain requirements of arc welding source. |  |
| c) | With neat diagram explain butt-welding. State its application. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any four requirements of ideal traction system. State and explain any four advantages of electrical traction. |  |
| b) | With neat diagram explain bridge transition method. |  |
| c) | Write short note on ‘Catenary construction’ for electric traction. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is stroboscopic effect? With an example explain how it can be reduced. |  |
| b) | With neat diagram explain working of HPMV lamp. |  |
| c) | Explain principle of production of light in i) Filament lamp ii) Gas discharge lamp. |  |

***********