## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-I(EE302) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the function of a DC generator? Briefly explain working principle of a simple loop DC generator. |  |
| b) | With neat diagram explain the classification of DC generators with respect to type of excitation system. |  |
| c) | Explain necessity of a starter for DC motor starting. |  |
| d) | A DC series motor should not be started on No load or light load. Explain with reasons. |  |
| e) | State the definition of a transformer and explain its working principle. |  |
| f) | List the conditions necessary to be satisfied for parallel operation of 3 phase transformers. |  |
| g) | With neat diagram explain the concept of open-delta connected transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the functions of any six parts of a d.c. machine. |  |
| b) | Explain the process of voltage build up in a separately excited de shunt generator. Draw the relevant characteristics and state two conditions necessary for its successful voltage build up. |  |
| c) | Explain the various types of losses taking place in a d.c. generator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the following characteristics for a dc series motor: i) Speed v/s armature current ii) Torque v/s armature current iii) Speed v/s Torque. |  |
| b) | With a neat diagram explain the armature control method for speed control of a dc shunt motor. State its advantages and disadvantages. |  |
| c) | With a neat labelled diagram of a transformer, briefly explain the construction and functions of any four parts of a transformer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between core type and shell type of construction of a transformer. Write atleast 6 points of distinction. |  |
| b) | A 50 KVA, 1, 50 Hz transformer has 500 turns on primary and 50 turns on secondary winding. The primary is connected to 3000V, 50 Hz supply. The maximum flux density is 1.3 Tesla. Find: i) Full load primary and secondary winding currents 2) Secondary emf 3) maximum flux in the core and 4) cross sectional area of the core. |  |
| c) | Explain the significance of equivalent circuit of a transformer. With neat circuit diagram explain the procedure for deducing the final equivalent circuit for a single phase practical transformer. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 30 KVA, 2400/120V, 50 Hz, 1 transformer has HV winding resistance of 0.1Ω and leakage reactance of 0.22Ω. The LV winding resistance is 0.035Ω and leakage reactance is 0.012Ω. Find equivalent winding resistance, reactance and impedance referred to: i) HV side and ii) LV side. |  |
| b) | A 5 KVA 200/400 V, 1, 50 Hz transformer has equivalent resistance and reactance referred to LV side as 0.5Ω and 1.5Ω respectively. Find terminal voltage on HV side when the transformer supplies 3/4th full load at 0.8 p.f. and supply voltage being 200V. Hence find its output and % efficiency at 3/4th full load if core losses are 120W. |  |
| c) | With neat diagrams explain the procedure and significance of conducting O.C. & S.C. test on a 1 transformer. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the significance and procedure for connecting two 1 transformer in parallel. State any 3 conditions necessary for their parallel operation. |  |
| b) | With a neat diagram explain the concept and working principle of an auto transformer. List its advantages over a two winding transformer. |  |
| c) | Write short notes on: |  |
|  | i) Welding transformer ii) Furnace transformer. |  |

***********