## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Auto Components(4067)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms i) Allowance ii) Tolerance iii) Standardization. |  |
| b) | A M.S. bolt is subjected to a tensile load of 4 KN. If the yield strength is 360 MPa, design the coarse threaded bolt taking a F.O.S. as 4. If the F.O.S. was increased to 6, would the diameter be bigger or smaller? |  |
| c) | Draw stress strain diagram and for a ductile M.S, how will you determine from the diagram i) Modulus of elasticity ii) Percentage elongation & Percentage reduction in area. |  |
| d) | The piston of a gas engine having diameter 300mm is driven by gas pressure of 0.7N/mm2. If the maximum permissible compressive stress for the piston rod is 40N/mm2, calculate its diameter. |  |
| e) | What are bolts of uniform strength? Describe them with the help of neat sketch. |  |
| f) | Using sketches explains splined keys and tangent keys and their specific uses. |  |
| g) | Explain what are inertia benders stresses for a connecting rod. If the speed is doubled, what would be the effect on stress? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | It is required to design a square key for fixing a gear on a shaft of 25mm diameter. 15KW of power is transmitted at 720 rpm from the shaft to the gear. The key is made of steel having of steel having 460 MPa strength is tension as well as compression. Assume F.O.S.=3 & design the key. Take Sys =0.5Syt. |  |
| b) | A rigid coupling is used to transmit 20KW at 720 rpm. There are 4 bolts and the P.C.D. of bolts is 125mm. The bolts are made of steel 45C8 (Syt=380N/mm2) & F.O.S.=3. Determine the diameters of the bolts taking Sys=0.577Syt. |  |
|  |  | ….2/- |
|  | -2- |  |
| c) | A gear box of a motor vehicle delivers a men torque of 6 KNm The Maximum torque is expected to be 20% more than the mean. An universal coupling delivers this torque to the output shaft. Design its shafts and pins. The allowable shear stresses for the shaft and pin may be taken as 55 N/mm2 and 30N/mm2 respectively. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - With a neat sketch explain the various styles of ends adopted for closed coil helical springs and the relation between total number of turns and the number of active turns for each style. ii) List various types of springs. State the material for helical springs. What are solid length and free length for helical coil spring? |  |
| b) | Design a helical compression spring for maximum load of 1000N and a deflection of 25mm using the value of spring index as 5. The maximum permissible shear stress for springs wire is 420N/mm2 and modulus of rigidity is 84 KN/mm2. |  |
| c) | A truck spring has 12 leaves, two of which are full length leaves. The spring supports are 1.5m apart and the central band is 85mm wide. The central load is 5.4 KN with a permissible stress of 280MPa. Determine thickness and width of spring leaves. The ratio of total depth to the width of the spring is 3. Also determine the deflection of the spring. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A force of 15KN is acting on C-frame as shown in fig.1. It is made of grey cast iron FG300 (Sut=300MPa) and the F.O.S. =2.5. Determine the dimension of the cross section of the frame. |  |
| b) | A 50 mm diameter rod supports a 9 KN load in addition to a torque of 100N-m as shown in fig.2. Determine the maximum tensile and maximum shear stress induced. |  |
| c) | A link as shown in fig.3 of is made of grey cast iron. It transmit a full P of 10 KN. Assuming the link has a square cross-section (b=h) and using a factor of safety of 5, determine the dimensions of cross-section of the link. What will be dimension of the link if it is made hollow and circular having outer diameter twice the inside diameter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A wall bracket shown in fig.4 is fixed to the walls by means of 4 bolts. A load of 20KN acts on the arm of the bracket at 500mm from the wall. Find the size of the bolts. Permissible stresses in tension and shear are 70MPa and 40MPa respectively. | …..3/- |
|  | -3- |  |
| b) | Determine the dimensions of an I-section of a connecting rod having the following specifications: Diameter of piston = 120mm, Stroke = 130mm, Mass of reciprocating parts = 3.45Kg Length of connecting rod = 350mm R.P.M=1800, Maximum explosion pressure = 3MPa Assume ultimate crushing stress for the material as 330 MPa and F.O.S.= 6. |  |
| c) | The following are the specifications for 4-stroke petrol engine. Piston diameter =120mm, Stroke = 140mm, Maximum speed 40 rev/sec Maximum explosion pressure = 2.45 MPa Length of connecting rod from centre to centre = 315mm, Mass of reciprocating parts 1.9Kg. Design the diameter of the two big end bolts and the thickness of the cap given that the distance between the centres of the two bolts as 62mm and the length of the big end bearing as 52mm. Take allowable tensile stresses for both materials as 120MPa. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design the crown thickness on the basis of strength and also find the wrist pin dimensions for a trunk type CI piston for an IC engine from following data. Diameter of cylinder = 100mm, Stroke = 150mm, Maximum explosion pressure = 3.5 N/mm2. For piston material ft=30MPa and for pin material flexural stress =110N/mm2. Also permissible crushing stress for small end bush material = 21N/mm2. |  |
| b) | A single plate clutch effective on both sides is engaged by means of 6 helical springs. The efficient of friction is 0.3 and the mean diameter of the friction surface is 228mm. Find the axial force exerted by each spring if the torque transmitted by the clutch is 245 N-m. What is the power transmitted by this clutch at 3600 rpm? If the power is to be increased what factors will you change? |  |
| c) | Determine the diameter of the hollow push rod made of M.S. for an IC Engine, if the max force exerted by the push rod is 1500N. The length of the push rod is 50 cm. Take F.O.S = 2.5 and E=210KN/mm2 and equivalent length push rod as 80% of its full length considering its both ends as not purely spherical seated. |  |

……4/-